# HTTP API说明书

# HTTP API Specifications

# VER. 1.5

修订记录

| 修订日期 | 修订版本 | 描述 | 修订人 |
| --- | --- | --- | --- |
| 2023-11-03 | V1.0 | 首次发布 | 李光明 |
| 2023-12-05 | V1.1 | 增加IO状态控制 | 李光明 |
| 2024-01-03 | V1.2 | 增加对补光灯亮度模式的配置支持 | 李光明 |
| 2024-01-15 | V1.3 | 1)事件通知增加Picture可选项可携带图片 2)事件通知增加OccurFlag属性，true表示报警发生，false表示报警结束 | 李光明 |
| 2024-01-16 | V1.4 | 1)事件通知增加本地时间字符串 2)补充设置目标检测HAPI方法章节 2)事件通知增加OccurFlag属性，true表示报警发生，false表示报警结束 | 李光明 |
| 2024-03-18 | V1.5 | 增加视频遮挡、人脸检测、越界检测、区域（周界）侦测、烟火检测配置GET/SET | 李光明 |

目录

HTTP API说明书	1

HTTP API Specifications	1

VER. 1.5	1

1	引言	7

1.1	HAPI概述	7

1.2	HAPI采用的HTTP方法	7

1.3	HAPI设备回应消息	7

1.4	HAPI URL规则	8

1.5	HAPI认证	8

1.5.1	用户名/密码认证方式	9

1.5.2	Session ID认证方式	9

1.6	HAPI接口示例代码	9

1.6.1	GET方法明文密码调用getuid	9

1.6.2	PUT方法调用getuid	9

1.6.3	GET方法密文密码调用getuid	10

1.6.4	GET方法调用keep_alive	10

1.6.5	PUT方法调用keep_alive	10

2	接口介绍	11

2.1	Session ID	11

2.1.1	获取uid /HAPI/V1.0/uid/getuid	11

2.1.2	保活uid /HAPI/V1.0/uid/keep_alive	11

2.2	系统信息	12

2.2.1	获取设备信息 /HAPI/V1.0/sysinfo/device_info	12

2.2.2	获取API接口列表 /HAPI/V1.0/sysinfo/functionlist	13

2.2.3	获取设备能力集 /HAPI/V1.0/sysinfo/capability	16

2.2.4	获取RTSP URL /HAPI/V1.0/sysinfo/rtspurl	18

2.3	系统控制	19

2.3.1	重启设备 /HAPI/V1.0/sysman/reboot	19

2.3.2	恢复出厂 /HAPI/V1.0/sysman/factory	19

2.4	IO状态控制	20

2.4.1	获取IO输入状态 /HAPI/V1.0/io/input/get	20

2.4.2	获取IO输出状态 /HAPI/V1.0/io/output/get	20

2.4.3	设置IO输出状态 /HAPI/V1.0/io/output/set	21

2.5	手动抓图	22

2.5.1	手动抓图 /HAPI/V1.0/snapshot.cgi	22

2.6	云台控制	23

2.6.1	云台停止 /HAPI/V1.0/ptz_ctrl/stop	23

2.6.2	云台转动 /HAPI/V1.0/ptz_ctrl/move	23

2.6.3	云台预置点操作 /HAPI/V1.0/ptz_ctrl/preset	24

2.6.4	云台变倍操作 /HAPI/V1.0/ptz_ctrl/zoom	25

2.6.5	云台聚焦操作 /HAPI/V1.0/ptz_ctrl/focus	25

2.6.6	云台光圈操作 /HAPI/V1.0/ptz_ctrl/iris	26

2.6.7	云台高级功能执行操作 /HAPI/V1.0/ptz_ctrl/advfunction/exec	27

2.6.8	云台高级功能列表 /HAPI/V1.0/ptz_ctrl/advfunction/get	27

2.7	系统时间	28

2.7.1	获取设备时间和配置 /HAPI/V1.0/systime/gettime	28

2.7.2	设置设备时间和时区 /HAPI/V1.0/systime/settime	29

2.7.3	设置设备NTP配置 /HAPI/V1.0/systime/setntp	29

2.8	灯光配置	30

2.8.1	获取支持的灯光开关控制模式能力集/HAPI/V1.0/ system/light/ctrlmode/capability	30

2.8.2	获取支持的灯光工作模式能力集/HAPI/V1.0/ system/light/workmode/capability	31

2.8.3	获取灯光配置/HAPI/V1.0/system/light/get	32

2.8.4	设置灯光配置/HAPI/V1.0/system/light/set	33

2.9	图像配置	34

2.9.1	获取图像配置/HAPI/V1.0/system/image/get	34

2.9.2	设置图像配置/HAPI/V1.0/system/image/set	35

2.10	视频配置	35

2.10.1	获取视频编码能力集/HAPI/V1.0/system/video/capability	35

2.10.2	获取视频编码配置/HAPI/V1.0/system/video/get	37

2.10.3	设置视频编码配置/HAPI/V1.0/system/video/set	39

2.11	音频配置	40

2.11.1	获取音频配置编码能力集/HAPI/V1.0/system/audio/capability	40

2.11.2	获取音频配置/HAPI/V1.0/system/audio/get	41

2.11.3	设置音频配置/HAPI/V1.0/system/audio/set	42

2.12	OSD配置	43

2.12.1	获取OSD配置/HAPI/V1.0/system/osd/get	43

2.12.2	设置OSD配置/HAPI/V1.0/system/osd/set	45

2.13	智能检测配置	47

2.13.1	获取智能检测能力集/HAPI/V1.0/smart/capability	47

2.13.2	获取目标检测（人车非机动车）算法支持的目标类型/HAPI/V1.0/smart/objectdetect /capability	48

2.13.3	获取目标检测（人车非机动车）算法支持联动能力/HAPI/V1.0/smart/linkage/capability	49

2.13.4	获取警戒音列表/HAPI/V1.0/smart/audiofiles/get	50

2.13.5	获取运动侦测配置/HAPI/V1.0/motiondetect/get	51

2.13.6	设置运动侦测配置/HAPI/V1.0/motiondetect/set	55

2.13.7	获取目标检测配置/HAPI/V1.0/smart/objectdetect/get	57

2.13.8	设置目标检测配置/HAPI/V1.0/smart/objectdetect/set	61

2.13.9	获取视频遮挡配置/HAPI/V1.0/smart/videocover/get	63

2.13.10	设置视频遮挡配置/HAPI/V1.0/smart/videocover/set	64

2.13.11	获取人脸检测配置/HAPI/V1.0/smart/facedetect/get	65

2.13.12	设置人脸检测配置/HAPI/V1.0/smart/facedetect/set	67

2.13.13	获取越界检测配置/HAPI/V1.0/smart/videogate/get	69

2.13.14	设置越界检测配置/HAPI/V1.0/smart/videogate/set	74

2.13.15	获取区域(周界)侦测配置/HAPI/V1.0/smart/regionai/get	75

2.13.16	设置区域(周界)侦测配置/HAPI/V1.0/smart/regionai/set	79

2.13.17	获取车牌识别配置/HAPI/V1.0/smart/lpr/get	80

2.13.18	设置车牌识别配置/HAPI/V1.0/smart/lpr/set	82

2.13.19	获取烟火检测配置/HAPI/V1.0/smart/flameflumes/get	84

2.13.20	设置烟火检测配置/HAPI/V1.0/smart/flameflumes/set	87

3	事件	89

3.1	事件工作流程	89

3.2	事件订阅	90

3.2.1	注册订阅/HAPI/V1.0/Event/subscription/regist	90

3.2.2	刷新订阅/HAPI/V1.0/Event/subscription/refresh	91

3.2.3	删除订阅/HAPI/V1.0/Event/subscription/delete	93

3.3	事件通知	94

3.3.1	事件通知/HAPI/V1.0/Event/Notification	94

4	JSON结构化数据说明	97

4.1	布防方式（ArmingMode）	97

4.2	报警检测布防配置	97

4.3	运动侦测区域配置	98

4.4	报警联动布防配置	99

4.5	警戒音联动配置	100

4.6	IO输出联动配置	100

4.7	目标检测类型	101

4.8	目标检测区域配置	101

4.9	矩形检测区域配置	102

4.10	越界检测规则配置	102

4.11	区域侦测规则配置	104

4.12	事件通知Json Block	105

4.12.1	MotionDetectInfo	105

4.12.1	ObjectDetectInfo	107

引言

HAPI概述

HTTP API简称为HAPI，采用HTTP短连接机制，一次交互过程中，客户端通过HTTP URL通知服务端需要请求的资源，服务端收到消息后，将URL指定的资源信息和请求结果等按照Json格式返回。

HAPI支持：

- 获取设备能力，以及支持的HAPI方法集合

- 系统控制，例如重启、恢复出厂等

- 云台控制，例如转动、预置点设置、预置点调用等

- 配置的获取和设置，例如网络、视频、音频、存储、报警、智能分析、云台等配置

- 事件订阅机制

- 事件报警上报

HAPI采用的HTTP方法

HAPI接口使用的http方法包括GET/PUT/POST等。

GET请求的消息体为空，需要的参数在URL中携带。URL以？开头携带参数，参数之间用&分割。

PUT/POST请求参数在消息体中以json格式携带。

POST请求需要指定Content-Type: application/x-www-form-urlencoded。

为了方便浏览器直接调试HAPI接口，大部分获取信息、系统控制、系统控制等HAPI接口支持GET方法，在URL中填写必要参数。支持GET的接口同时支持PUT方法，在消息体中用json格式携带多项参数。

事件订阅、刷新、取消订阅、通知都采用POST方法

HAPI设备回应消息

当方法是GET时，HTTP请求在URL中携带需要的参数，消息体为空。

当方法是PUT

POST时，HTTP消息体通常需要包含JSON数据。请求报文必须发往设备的HTTP/HTTPS端口。

设备以JSON数据格式进行回应，通常包含如下字段，各字段含义如下：

ResponseURL：表示HAPI请求报文URL请求的方法

SessionID：getuid返回的接口Session ID。如果是用户名/密码认证方式，SessionID为空值。

ResponseCode：表示系统处理结果，0表示成功

ResponseString：Succeed表示成功，其他文本为为错误提示

Data：表示请求报文（GET）的响应结果，多数情况下是JSON格式。若URL相同，PUT请求报文数据格式与GET响应报文中的数据格式相同。下表中的Data项列出了每个API的数据。如果服务器处理失败，Data为“null”。如果HTTP报文方法是PUT/POST，响应报文中的Data为“null”。

{

"Response": {

"ResponseURL": "/HAPI/V1.0/uid/getuid",

" SessionID": "15E25D",

"ResponseCode": 0,

"ResponseString": "Succeed",

"Data": "null",

}

}

HAPI URL规则

/HAPI/V1.0[/Channels/<ID>]/<service-name>/<resource-name>[/<child-resource-name>][/<ID>]][?<key1>=<value1>[&<key2>=<value2>]]

参数说明：

| 组成 | 说明 |
| --- | --- |
| [ ]: | 表示非必填参数 |
| <> | 表示必填参数 |
| [/Channels/<ID>] | 通道号，多通道设备ID为0表示通道号无关，ID为从1开始表示通道号（适用于NVR和多通道摄像机）。单目摄像机该参数可省略。 |
| <service-name> | 业务类型，比如系统、媒体、网络、存储、云台控制等 |
| <resource-name> | 资源名称 |
| [/<child-resource-name>] | 子资源名称，非必填 |
| [?<key1>=<value1>[&<key2>=<value2>]] | 以？开头，携带需要的参数。多个参数之间用&分割 |

HAPI认证

设备收到HAPI请求时，首先会对请求的API URL进行合法性认证。合法性认证不通过的请求，回应失败信息给请求端。

HTTP请求携带Session ID或者用户名/密码作为认证信息，GET请求在URL中携带，PUT/POST请求在消息体中携带。

用户名/密码认证方式

直接携带用户名和密码信息，密码支持明文和32位MD5加密字符串：

[username=<username>&password=<password>]

用户名/密码字符串的大小写不影响认证结果。

Session ID认证方式

首先通过/HAPI/V1.0/uid/getuid接口获取Session ID，携带上节所述的用户名和密码信息。后续调用HAPI接口都需要在携带该Session ID。

Session ID在最后一次成功请求后的 60秒超时，失效之后需要重新请求Session ID。如果长时间没有HAPI请求但又希望Session ID不失效，可以使用/HAPI/V1.0/uid/ keep_alive保持心跳。

[uid=<uid>]

HAPI接口示例代码

GET方法明文密码调用getuid

GET /HAPI/V1.0/uid/getuid?username=admin&password=123456

返回结果：

{

"Response": {

"ResponseURL": "/HAPI/V1.0/uid/getuid",

"SessionID": "371C1A3",

"ResponseCode": 0,

"ResponseString": "Succeed",

"Data": "null"

}

}

PUT方法调用getuid

PUT /HAPI/V1.0/uid/getuid HTTP/1.1

User-Agent: PostmanRuntime-ApipostRuntime/1.1.0

Cache-Control: no-cache

content-type: application/json

Accept: */*

Accept-Encoding: gzip, deflate, br

Connection: keep-alive

Host: 192.168.1.202

Content-Length: 82

{

"username":"admin",

"password":"e10adc3949ba59abbe56e057f20f883e"

}

返回结果：

{

"Response": {

"ResponseURL": "/HAPI/V1.0/uid/getuid",

"SessionID": "3729116",

"ResponseCode": 0,

"ResponseString": "Succeed",

"Data": "null"

}

}

GET方法密文密码调用getuid

GET /HAPI/V1.0/uid/getuid?username=admin&password=e10adc3949ba59abbe56e057f20f883e

GET方法调用keep_alive

GET /HAPI/V1.0/uid/keep_alive?uid=15E25D

PUT方法调用keep_alive

PUT /HAPI/V1.0/uid/keep_alive HTTP/1.1

User-Agent: PostmanRuntime-ApipostRuntime/1.1.0

Cache-Control: no-cache

content-type: application/json

Accept: */*

Accept-Encoding: gzip, deflate, br

Connection: keep-alive

Host: 192.168.1.202

Content-Length: 25

{

"uid":"3CFABD6"

}

接口介绍

Session ID

获取uid /HAPI/V1.0/uid/getuid

| URL | /HAPI/V1.0/uid/getuid |
| --- | --- |
| Description | 获取uid，用于后续API会话 |
| Method | GET/PUT |
| Sample | http://192.168.1.202/HAPI/V1.0/uid/getuid?username=admin&password=e10adc3949ba59abbe56e057f20f883e |
| URL params(简单参数) Input Data(PUT) | username/password |
| Success Return Data | { "Response": { "ResponseURL": "/HAPI/V1.0/uid/getuid", "SessionID": "3CC2457", "ResponseCode": 0, "ResponseString": "Succeed", "Data": "null" } } |
| Note |  |
| Status |  |

保活uid /HAPI/V1.0/uid/keep_alive

| URL | /HAPI/V1.0/uid/keep_alive |
| --- | --- |
| Description | 定期（60秒以内）发送心跳，使改uid在设备上一直有效 |
| Method | GET/PUT |
| Sample | http://192.168.1.202/HAPI/V1.0/uid/keep_alive?uid=3CFABD6 |
| URL params(简单参数) Input Data(PUT) | uid |
| Success Return Data | { "Response": { "ResponseURL": "/HAPI/V1.0/uid/keep_alive", "SessionID": "3CC2457", "ResponseCode": 0, "ResponseString": "Succeed", "Data": "null" } } |
| Note |  |
| Status |  |

系统信息

获取设备信息 /HAPI/V1.0/sysinfo/device_info

| URL | /HAPI/V1.0/sysinfo/device_info |
| --- | --- |
| Description | 获取设备信息 |
| Method | GET/PUT |
| Sample | http://192.168.1.202/HAPI/V1.0/sysinfo/device_info?username=admin&password=123456 |
| URL params(简单参数) Input Data(PUT) | username/password或者uid |
| Success Return Data | { "Response": { "ResponseURL": "/HAPI/V1.0/sysinfo/device_info", "SessionID": "", "ResponseCode": 0, "ResponseString": "Succeed", "Data": { "SN": "EF000000000000A3", "device_type": "YM800S", "model": "MC800S", "ether": "F0:00:00:00:00:A3", "kernelversion": "Linux 4.9.84 #73 SMP PREEMPT Mon Feb 28 12:36:32 CST 2022 armv7l", "fsversion": "YM800S_AF_V0_Y_EN-RTMP-H5_L V3.2.5.3 build 2023-10-31 15:04:32 " } } } |
| Note |  |
| Status |  |

获取API接口列表 /HAPI/V1.0/sysinfo/functionlist

| URL | /HAPI/V1.0/sysinfo/functionlist |
| --- | --- |
| Description | 获取设备支持的API接口列表 |
| Method | GET/PUT |
| Sample | http://192.168.1.202/HAPI/V1.0/sysinfo/functionlist?username=admin&password=e10adc3949ba59abbe56e057f20f883e |
| URL params(简单参数) Input Data(PUT) | username/password或者uid |
| Success Return Data | { "Response": { "ResponseURL": "/HAPI/V1.0/sysinfo/functionlist", "SessionID": "", "ResponseCode": 0, "ResponseString": "Succeed", "Data": [ { "api": "/HAPI/V1.0[/Channels/ID]/uid/getuid" }, { "api": "/HAPI/V1.0[/Channels/ID]/uid/keep_alive" }, { "api": "/HAPI/V1.0[/Channels/ID]/sysinfo/device_info" }, { "api": "/HAPI/V1.0[/Channels/ID]/sysinfo/functionlist" }, { "api": "/HAPI/V1.0[/Channels/ID]/sysinfo/capability" }, { "api": "/HAPI/V1.0[/Channels/ID]/sysinfo/rtspurl" }, { "api": "/HAPI/V1.0[/Channels/ID]/sysman/reboot" }, { "api": "/HAPI/V1.0[/Channels/ID]/sysman/factory" }, { "api": "/HAPI/V1.0[/Channels/ID]/io/input/get" }, { "api": "/HAPI/V1.0[/Channels/ID]/io/output/get" }, { "api": "/HAPI/V1.0[/Channels/ID]/io/output/set" }, { "api": "/HAPI/V1.0[/Channels/ID]/systime/gettime" }, { "api": "/HAPI/V1.0[/Channels/ID]/systime/settime" }, { "api": "/HAPI/V1.0[/Channels/ID]/systime/setntp" }, { "api": "/HAPI/V1.0[/Channels/ID]/ptz_ctrl/stop" }, { "api": "/HAPI/V1.0[/Channels/ID]/ptz_ctrl/move" }, { "api": "/HAPI/V1.0[/Channels/ID]/ptz_ctrl/preset" }, { "api": "/HAPI/V1.0[/Channels/ID]/ptz_ctrl/zoom" }, { "api": "/HAPI/V1.0[/Channels/ID]/ptz_ctrl/focus" }, { "api": "/HAPI/V1.0[/Channels/ID]/ptz_ctrl/iris" }, { "api": "/HAPI/V1.0[/Channels/ID]/ptz_ctrl/advfunction/exec" }, { "api": "/HAPI/V1.0[/Channels/ID]/ptz_ctrl/advfunction/get" }, { "api": "/HAPI/V1.0[/Channels/ID]/system/light/ctrlmode/capability" }, { "api": "/HAPI/V1.0[/Channels/ID]/system/light/workmode/capability" }, { "api": "/HAPI/V1.0[/Channels/ID]/system/light/get" }, { "api": "/HAPI/V1.0[/Channels/ID]/system/light/set" }, { "api": "/HAPI/V1.0[/Channels/ID]/system/image/get" }, { "api": "/HAPI/V1.0[/Channels/ID]/system/image/set" }, { "api": "/HAPI/V1.0[/Channels/ID]/system/video/capability" }, { "api": "/HAPI/V1.0[/Channels/ID]/system/video/get" }, { "api": "/HAPI/V1.0[/Channels/ID]/system/video/set" }, { "api": "/HAPI/V1.0[/Channels/ID]/system/audio/capability" }, { "api": "/HAPI/V1.0[/Channels/ID]/system/audio/get" }, { "api": "/HAPI/V1.0[/Channels/ID]/system/audio/set" }, { "api": "/HAPI/V1.0[/Channels/ID]/system/osd/get" }, { "api": "/HAPI/V1.0[/Channels/ID]/system/osd/set" }, { "api": "/HAPI/V1.0[/Channels/ID]/Smart/capability" }, { "api": "/HAPI/V1.0[/Channels/ID]/Smart/audiofiles/get" }, { "api": "/HAPI/V1.0[/Channels/ID]/Smart/objectdetect/capability" }, { "api": "/HAPI/V1.0[/Channels/ID]/Smart/linkage/capability" }, { "api": "/HAPI/V1.0[/Channels/ID]/Smart/motiondetect/get" }, { "api": "/HAPI/V1.0[/Channels/ID]/Smart/motiondetect/set" }, { "api": "/HAPI/V1.0[/Channels/ID]/Smart/objectdetect/get" }, { "api": "/HAPI/V1.0[/Channels/ID]/Smart/objectdetect/set" }, { "api": "/HAPI/V1.0[/Channels/ID]/Event/subscription/regist" }, { "api": "/HAPI/V1.0[/Channels/ID]/Event/subscription/refresh" }, { "api": "/HAPI/V1.0[/Channels/ID]/Event/subscription/delete" } ] } } |
| Note | API接口列表通过json数组返回 |
| Status |  |

获取设备能力集 /HAPI/V1.0/sysinfo/capability

| URL | /HAPI/V1.0/sysinfo/capability |
| --- | --- |
| Description | 获取设备能力集 |
| Method | GET/PUT |
| Sample | http://192.168.1.202/HAPI/V1.0/sysinfo/capability?username=admin&password=e10adc3949ba59abbe56e057f20f883e |
| URL params(简单参数) Input Data(PUT) | username/password或者uid |
| Success Return Data | { "Response": { "ResponseURL": "/HAPI/V1.0/sysinfo/capability", "SessionID": "", "ResponseCode": 0, "ResponseString": "Succeed", "Data": [ { "caps": "gpio_input" }, ………… { "caps": "dzoomsetting" } ] } } |
| Note |  |
| Status |  |

灯光、智能、编码等需要关注的能力集字符串列表

| ptz_control | 具备云台控制能力 |
| --- | --- |
| audio_action | 支持报警联动声音输出 |
| ledtype_set | 支持设置补光灯模式：智能双光/纯红外/纯白光 |
| ir_led_panel | 纯红外灯板，补光灯模式不支持切换到白光/双光模式 |
| white_led_panel | 纯白光灯板，补光灯模式不支持切换到白光/双光模式 |
| double_led_panel | 智能双光灯板 |
| ircut_setting | 支持设置IRCUT模式（软光敏自动、外部、定时） |
| ircut_ledmanualswitch | IRCUT模式增加2种：手动灯光常开、手动灯光常关(具体什么灯由补光灯模式决定) |
| ircut_leddelay | 支持设置软光敏的开关灯灵敏度 |
| AlarmVG | 拌线检测 |
| AlarmRegionAI | 区域侦测 |
| VideoPD | 人车检测 |
| face_detect | 人脸检测 |
| face_recognize | 人脸识别 |
| VehicleCar | 车形检测-汽车 |
| VehicleMoto | 车形检测-摩托 |
| VehicleElectricbicycle | 车型检测-电单车 |
| VehicleBicycle | 车型检测-自行车 |
| LPRecognition | 车牌识别 |
| Fire | 火焰检测 |
| three_video | 支持视频第三码流 |
| twolens_stitch | 双目拼接摄像机 |

获取RTSP URL /HAPI/V1.0/sysinfo/rtspurl

| URL | /HAPI/V1.0/sysinfo/rtspurl |
| --- | --- |
| Description | 获取RTSP URL |
| Method | GET/PUT |
| Sample | http://192.168.1.202/HAPI/V1.0/sysinfo/rtspurl?username=admin&password=e10adc3949ba59abbe56e057f20f883e |
| URL params(简单参数) Input Data(PUT) | username/password或者uid |
| Success Return Data | { "Response": { "ResponseURL": "/HAPI/V1.0/sysinfo/rtspurl", "SessionID": "", "ResponseCode": 0, "ResponseString": "Succeed", "Data": { "ch0_main": "rtsp://192.168.1.202:554/stream0", "ch0_sub": "rtsp://192.168.1.202:554/stream1" } } } |
| Note |  |
| Status |  |

系统控制

重启设备 /HAPI/V1.0/sysman/reboot

| URL | /HAPI/V1.0/sysman/reboot |
| --- | --- |
| Description | 重启设备 |
| Method | GET/PUT |
| Sample | http://192.168.1.202/HAPI/V1.0/sysman/reboot?username=admin&password=e10adc3949ba59abbe56e057f20f883e |
| URL params(简单参数) Input Data(PUT) | username/password或者uid |
| Success Return Data | { "Response": { "ResponseURL": "/HAPI/V1.0/sysman/reboot", "SessionID": "", "ResponseCode": 0, "ResponseString": "Succeed", "Data": "null" } } |
| Note |  |
| Status |  |

恢复出厂 /HAPI/V1.0/sysman/factory

| URL | /HAPI/V1.0/sysman/ factory |
| --- | --- |
| Description | 恢复出厂 |
| Sample | http://192.168.1.202/HAPI/V1.0/sysman/factory?username=admin&password=e10adc3949ba59abbe56e057f20f883e |
| URL params(简单参数) Input Data(PUT) | username/password或者uid |
| Method | GET/PUT |
| Input Data | 无 |
| Success Return Data | { "Response": { "ResponseURL": "/HAPI/V1.0/sysman/factory", "SessionID": "", "ResponseCode": 0, "ResponseString": "Succeed", "Data": "null" } } |
| Note |  |
| Status |  |

IO状态控制

获取IO输入状态 /HAPI/V1.0/io/input/get

| URL | /HAPI/V1.0/io/input/get |
| --- | --- |
| Description | 获取IO输入状态 |
| Method | GET/PUT |
| Sample | http://192.168.1.202/HAPI/V1.0/io/input/get?chn=1&username=admin&password=e10adc3949ba59abbe56e057f20f883e |
| URL params(简单参数) Input Data(PUT) | chn：IO通道号，从1开始，根据IPC实际配置 username/password或者uid |
| Success Return Data | { "Response": { "ResponseURL": "/HAPI/V1.0/io/input/get", "SessionID": "", "ResponseCode": 0, "ResponseString": "Succeed", "Data": 0 } } |
| Note | IO通道号不存在时，返回错误： { "Response": { "ResponseURL": "/HAPI/V1.0/io/input/get", "SessionID": "", "ResponseCode": -1, "ResponseString": "params error", "Data": "null" } } |
| Status |  |

获取IO输出状态 /HAPI/V1.0/io/output/get

| URL | /HAPI/V1.0/io/output/get |
| --- | --- |
| Description | 获取IO输出状态 |
| Method | GET/PUT |
| Sample | http://192.168.1.202/HAPI/V1.0/io/output/get?chn=1&username=admin&password=e10adc3949ba59abbe56e057f20f883e |
| URL params(简单参数) Input Data(PUT) | chn：IO通道号，从1开始，根据IPC实际配置 username/password或者uid |
| Success Return Data | { "Response": { "ResponseURL": "/HAPI/V1.0/io/output/get", "SessionID": "", "ResponseCode": 0, "ResponseString": "Succeed", "Data": 1 } } |
| Note | IO通道号不存在时，返回错误： { "Response": { "ResponseURL": "/HAPI/V1.0/io/output/get", "SessionID": "", "ResponseCode": -1, "ResponseString": "params error", "Data": "null" } } |
| Status |  |

设置IO输出状态 /HAPI/V1.0/io/output/set

| URL | /HAPI/V1.0/io/output/set |
| --- | --- |
| Description | 获取IO输出状态 |
| Method | GET/PUT |
| Sample | http://192.168.1.202/HAPI/V1.0/io/output/set?chn=1&status=0&username=admin&password=e10adc3949ba59abbe56e057f20f883e |
| URL params(简单参数) Input Data(PUT) | chn：IO通道号，从1开始，根据IPC实际配置 username/password或者uid |
| Success Return Data | { "Response": { "ResponseURL": "/HAPI/V1.0/io/output/set", "SessionID": "", "ResponseCode": 0, "ResponseString": "Succeed", "Data": 0 } } |
| Note | IO通道号不存在时，返回错误： { "Response": { "ResponseURL": "/HAPI/V1.0/io/output/set", "SessionID": "", "ResponseCode": -1, "ResponseString": "params error", "Data": "null" } } |
| Status |  |

手动抓图

手动抓图 /HAPI/V1.0/snapshot.cgi

| URL | /HAPI/V1.0/snapshot.cgi |
| --- | --- |
| Description | 获取图片 |
| Sample | http://192.168.1.202/HAPI/V1.0/snapshot.cgi?stream=1&username=admin&password=e10adc3949ba59abbe56e057f20f883e http://192.168.1.202/HAPI/V1.0/snapshot.cgi?stream=0&username=admin&password=e10adc3949ba59abbe56e057f20f883e |
| URL params(简单参数) Input Data(PUT) | Stream：抓图码流序号，stream=0表示抓子码流，stream=1表示抓主码流； username/password或者uid |
| Method | GET/PUT |
| Input Data | 无 |
| Success Return Data | Jpg图片 |
| Note |  |
| Status |  |

云台控制

云台停止 /HAPI/V1.0/ptz_ctrl/stop

| URL | /HAPI/V1.0/ptz_ctrl/stop |
| --- | --- |
| Description | 停止控制云台 |
| Method | GET/PUT |
| Sample | http://192.168.1.202/HAPI/V1.0/ptz_ctrl/stop?username=admin&password=e10adc3949ba59abbe56e057f20f883e |
| URL params(简单参数) Input Data(PUT) | username/password或者uid |
| Success Return Data | { "Response": { "ResponseURL": "/HAPI/V1.0/ptz_ctrl/stop", "SessionID": "", "ResponseCode": 0, "ResponseString": "Succeed", "Data": "null" } } |
| Note |  |
| Status |  |

云台转动 /HAPI/V1.0/ptz_ctrl/move

| URL | /HAPI/V1.0/ptz_ctrl/move |
| --- | --- |
| Description | 控制云台转动方向，支持8向控制 |
| Method | GET/PUT |
| Sample | http://192.168.1.202/HAPI/V1.0/ptz_ctrl/move?direction=left&autostop=500&speed=5&username=admin&password=e10adc3949ba59abbe56e057f20f883e |
| URL params(简单参数) Input Data(PUT) | direction：云台移动方向，取值范围："left", "right", "up", "down", "left_up", "right_up", "left_down", "right_down"； speed：云台速度，取值范围1-10； autostop：控制后达到指定毫秒时自动停止，阻塞式调用，大于1000毫秒的数值将在1000毫秒是自动停止。不填或者为0时表示不自动停止 username/password或者uid |
| Success Response Data | { "Response": { "ResponseURL": "/HAPI/V1.0/ptz_ctrl/move", "SessionID": "", "ResponseCode": 0, "ResponseString": "Succeed", "Data": { "direction": "left", "speed": 5, "autostop": 500 } } } |
| Note |  |
| Status |  |

云台预置点操作 /HAPI/V1.0/ptz_ctrl/preset

| URL | /HAPI/V1.0/ptz_ctrl/preset |
| --- | --- |
| Description | 控制云台转动方向，支持8向控制 |
| Method | GET/PUT |
| Sample | http://192.168.1.202/HAPI/V1.0/ptz_ctrl/preset?method=set&presetno=1&username=admin&password=e10adc3949ba59abbe56e057f20f883e http://192.168.1.202/HAPI/V1.0/ptz_ctrl/preset?method=call&presetno=2&uid=3CFABD6 http://192.168.1.202/HAPI/V1.0/ptz_ctrl/preset?method=delete&presetno=3&uid=3CFABD6 |
| URL params(简单参数) Input Data(PUT) | method：预置点操作方法，取值范围："set", "call", "delete"； presetno：预置点序号，取值范围1-255； username/password或者uid |
| Success Response Data | { "Response": { "ResponseURL": "/HAPI/V1.0/ptz_ctrl/move", "SessionID": "", "ResponseCode": 0, "ResponseString": "Succeed", " Data": { "direction": "left", "speed": 5 } } } |
| Note |  |
| Status |  |

云台变倍操作 /HAPI/V1.0/ptz_ctrl/zoom

| URL | /HAPI/V1.0/ptz_ctrl/zoom |
| --- | --- |
| Description | 控制云台变倍 |
| Method | GET/PUT |
| Sample | 放大： http://192.168.1.202/HAPI/V1.0/ptz_ctrl/zoom?direction=in&autostop=500&username=admin&password=e10adc3949ba59abbe56e057f20f883e 缩小： http://192.168.1.202/HAPI/V1.0/ptz_ctrl/zoom?direction=out&autostop=100&username=admin&password=e10adc3949ba59abbe56e057f20f883e |
| URL params(简单参数) Input Data(PUT) | Direction：变倍方向，取值范围："in", "out"； autostop：控制后达到指定毫秒时自动停止，阻塞式调用，大于1000毫秒的数值将在1000毫秒是自动停止。不填或者为0时表示不自动停止 username/password或者uid |
| Success Response Data | { "Response": { "ResponseURL": "/HAPI/V1.0/ptz_ctrl/zoom", "SessionID": "", "ResponseCode": 0, "ResponseString": "Succeed", "Data": { "direction": "in", "autostop": 500 } } } |
| Note | Response中携带控制方向。 |
| Status |  |

云台聚焦操作 /HAPI/V1.0/ptz_ctrl/focus

| URL | /HAPI/V1.0/ptz_ctrl/focus |
| --- | --- |
| Description | 控制云台聚焦 |
| Method | GET/PUT |
| Sample | 向近处聚焦： http://192.168.1.202/HAPI/V1.0/ptz_ctrl/focus?direction=near&autostop=100&username=admin&password=e10adc3949ba59abbe56e057f20f883e 向远处聚焦： http://192.168.1.202/HAPI/V1.0/ptz_ctrl/focus?direction=far&autostop=100&username=admin&password=e10adc3949ba59abbe56e057f20f883e |
| URL params(简单参数) Input Data(PUT) | Direction：聚焦方向，取值范围："near", "far"； autostop：控制后达到指定毫秒时自动停止，阻塞式调用，大于1000毫秒的数值将在1000毫秒是自动停止。不填或者为0时表示不自动停止； username/password或者uid |
| Success Response Data | { "Response": { "ResponseURL": "/HAPI/V1.0/ptz_ctrl/focus", "SessionID": "", "ResponseCode": 0, "ResponseString": "Succeed", "Data": { "direction": "near", "autostop": 100 } } } |
| Note | Response中携带控制方向。 |
| Status |  |

云台光圈操作 /HAPI/V1.0/ptz_ctrl/iris

| URL | /HAPI/V1.0/ptz_ctrl/iris |
| --- | --- |
| Description | 控制云台光圈 |
| Method | GET/PUT |
| Sample | 光圈加大： http://192.168.1.202/HAPI/V1.0/ptz_ctrl/iris?direction=open&autostop=100&username=admin&password=e10adc3949ba59abbe56e057f20f883e 光圈缩小： http://192.168.1.202/HAPI/V1.0/ptz_ctrl/iris?direction=close&autostop=100&username=admin&password=e10adc3949ba59abbe56e057f20f883e |
| URL params(简单参数) Input Data(PUT) | direction：光圈控制方向，取值范围："open", "close"； autostop：控制后达到指定毫秒时自动停止，阻塞式调用，大于1000毫秒的数值将在1000毫秒是自动停止。不填或者为0时表示不自动停止； username/password或者uid |
| Success Response Data | { "Response": { "ResponseURL": "/HAPI/V1.0/ptz_ctrl/iris", "SessionID": "", "ResponseCode": 0, "ResponseString": "Succeed", "Data": { "direction": "open", "autostop": 100 } } } |
| Note | Response中携带控制方向。 |
| Status |  |

云台高级功能执行操作 /HAPI/V1.0/ptz_ctrl/advfunction/exec

| URL | /HAPI/V1.0/ptz_ctrl/advfunction/exec |
| --- | --- |
| Description | 执行云台高级功能 |
| Method | GET/PUT |
| Sample | http://192.168.1.202/HAPI/V1.0/ptz_ctrl/advfunction/exec?functionname=PTZReset&username=admin&password=e10adc3949ba59abbe56e057f20f883e |
| URL params(简单参数) Input Data(PUT) | functionname：高级功能名称，取值范围需要根据设备配置来确定； username/password或者uid |
| Success Response Data | { "Response": { "ResponseURL": "/HAPI/V1.0/ptz_ctrl/advfunction/exec", "SessionID": "", "ResponseCode": 0, "ResponseString": "Succeed", " Data": { "functionname": "PTZReset" } } } |
| Note | 设备可以配置一系列云台高级功能，每个高级功能执行某一个或多一个预置点的设置或调用。 |
| Status |  |

云台高级功能列表 /HAPI/V1.0/ptz_ctrl/advfunction/get

| URL | /HAPI/V1.0/ptz_ctrl/advfunction/get |
| --- | --- |
| Description | 获取设备云台高级功能列表 |
| Method | GET/PUT |
| Sample | http://192.168.1.202/HAPI/V1.0/ptz_ctrl/advfunction/get?username=admin&password=e10adc3949ba59abbe56e057f20f883e |
| URL params(简单参数) Input Data(PUT) | username/password或者uid |
| Success Response Data | { "Response": { "ResponseURL": "/HAPI/V1.0/ptz_ctrl/advfunction/get", "SessionID": "", "ResponseCode": 0, "ResponseString": "Succeed", "Data": [ { "functionname": "GuardPositionOff" }, ………… { "functionname": "TrackAutoOff" }, { "functionname": "TrackAutoOn" } ] } } |
| Note | 设备可以配置一系列云台高级功能，每个高级功能执行某一个或多一个预置点的设置或调用。本API用于获取已经配置的更高级功能列表。 |
| Status |  |

系统时间

获取设备时间和配置 /HAPI/V1.0/systime/gettime

| URL | /HAPI/V1.0/systime/gettime |
| --- | --- |
| Description | 获取设备当前时间和时间配置 |
| Method | GET/PUT |
| Sample | http://192.168.1.202/HAPI/V1.0/systime/gettime?username=admin&password=e10adc3949ba59abbe56e057f20f883e |
| URL params(简单参数) Input Data(PUT) | username/password或者uid |
| Success Return Data | { "Response": { "ResponseURL": "/HAPI/V1.0/systime/gettime", "SessionID": "", "ResponseCode": 0, "ResponseString": "Succeed", "Data": { "timeMode": "MANUAL", "timeZone": 1200, "nowtime": "2023-11-06 20:13:46" } } } |
| Note | timeMode: 校时方式，Manual，P2P，NTP timeZone：时区，整数，（时区+12）*60，例如东8区的timeZone值为1200 nowtime：设备本地时间字符串，按照年月日时分秒顺序格式化为"%04d-%02d-%02d %02d:%02d:%02d" |
| Status |  |

设置设备时间和时区 /HAPI/V1.0/systime/settime

| URL | /HAPI/V1.0/systime/settime |
| --- | --- |
| Description | 设置设备时间 |
| Method | GET/PUT |
| Sample | http://192.168.1.202/HAPI/V1.0/systime/settime?localtime=20231106202840&timeZone=1260&username=admin&password=e10adc3949ba59abbe56e057f20f883e |
| URL params(简单参数) Input Data(PUT) | timeZone：时区，整数，（时区+12）*60，例如东8区的timeZone值为1200，可缺省，不填的时候不改变设备时区； localtime：设置本地时间字符串，按照年月日时分秒顺序格式化为"%04d%02d%02d%02d%02d%02d"，可以缺省，不填的时候不改变设备时间； username/password或者uid |
| Success Return Data | 参见2.7.1GET返回数据。 |
| Note | " |
| Status |  |

设置设备NTP配置 /HAPI/V1.0/systime/setntp

| URL | /HAPI/V1.0/systime/setntp |
| --- | --- |
| Description | 设置设备NTP配置 |
| Method | GET/PUT |
| Sample | http://192.168.1.202/HAPI/V1.0/systime/setntp?serverIP=ipvs.icamra.com&username=admin&password=e10adc3949ba59abbe56e057f20f883e |
| URL params(简单参数) Input Data(PUT) | NTP serverIP/serverPort/refreshInterval，username/password或者uid |
| Success Return Data | 参见2.7.1GET返回数据。 |
| Note | serverPort可缺省，默认为123. refreshInterval可缺省，默认为60（秒） |
| Status |  |

灯光配置

获取支持的灯光开关控制模式能力集/HAPI/V1.0/ system/light/ctrlmode/capability

| URL | /HAPI/V1.0/system/light/ctrlmode/capability |
| --- | --- |
| Description | 获取支持的灯光开关控制模式列表 |
| Method | GET/PUT |
| Sample | http://192.168.1.202/HAPI/V1.0/system/light/ctrlmode/capability?username=admin&password=e10adc3949ba59abbe56e057f20f883e |
| URL params(简单参数) Input Data(PUT) | username/password或者uid |
| Success Response Data | { "Response": { "ResponseURL": "/HAPI/V1.0/system/light/ctrlmode/capability ", "SessionID": "", "ResponseCode": 0, "ResponseString": "Succeed", "Data": [ { "mode": 0 }, { "mode": 1 }, { "mode": 2 }, { "mode": 3 } ] } } |
| Note | 灯光开关控制模式（也叫IRCUT控制模式、日夜切换模式），用于配置开灯、切换IRCUT、以及图像彩转灰的控制模式，取值范围为： 0：主动模式/软光敏自动控制模式, ISP自动判断SENSOR增益来控制 1：日夜模式，根据时间段来控制 2: 被动模式/硬光敏外部控制模式，灯板根据自身光敏电阻给的硬件信号来开灯，并通过高低电平通知摄像机，摄像机根据该信号来切换IRCUT和图像彩转灰 3：手动模式，由调用者来手动切换，主要用于界面上测试IRCUT功能 4：反向被动模式，已废弃 5： 硬光敏自动控制模式，模组根据光敏电阻硬件的adc数据来控制 6：手动灯光常开 7：手动灯光常关 |
| Status |  |

获取支持的灯光工作模式能力集/HAPI/V1.0/ system/light/workmode/capability

| URL | /HAPI/V1.0/system/light/workmode/capability |
| --- | --- |
| Description | 获取支持的灯光开关控制模式列表 |
| Method | GET/PUT |
| Sample | http://192.168.1.202/HAPI/V1.0/system/light/workmode/capability?username=admin&password=e10adc3949ba59abbe56e057f20f883e |
| URL params(简单参数) Input Data(PUT) | username/password或者uid |
| Success Response Data | { "Response": { "ResponseURL": "/HAPI/V1.0/system/light/workmode/capability", "SessionID": "", "ResponseCode": 0, "ResponseString": "Succeed", "Data": [ { "mode": 0 }, { "mode": 1 }, { "mode": 2 } ] } } |
| Note | 灯光工作模式，用于配置开灯时补光灯的工作模式，取值范围： 0：纯红外，需要开灯的条件下只开红外灯 1：纯白光，需要开灯的条件下只开白光灯 2：智能双光，需要开灯的条件下，根据图像智能检测结果，有检测事件时开白光灯并保持彩色图像，没有检测事件时开红外灯 |
| Status |  |

获取灯光配置/HAPI/V1.0/system/light/get

| URL | /HAPI/V1.0/system/light/get |
| --- | --- |
| Description | 获取灯光配置 |
| Method | GET/PUT |
| Sample | http://192.168.1.202/HAPI/V1.0/system/light/get?username=admin&password=e10adc3949ba59abbe56e057f20f883e |
| URL params(简单参数) Input Data(PUT) | username/password或者uid |
| Success Response Data | { "Response": { "ResponseURL": "/HAPI/V1.0/system/light/get", "SessionID": "", "ResponseCode": 0, "ResponseString": "Succeed", "Data": { "led_ctrl_mode": 2, "led_work_mode": 0, "light_open_brightness": 3, "light_off_sensitivity": 50, " led_brightness_mode": 0, "led_brightness_value": 100, "night_starttime": "18:00:00", "night_endtime": "08:00:00" } } } |
| Note | led_ctrl_mode: 灯光开关控制模式，取值范围参见2.8.1 led_work_mode: 灯光工作模式，取值范围参见2.8.2； light_open_brightness: 开启环境光亮度，取值范围0-17，值越小说明环境亮度越低才开灯； light_off_sensitivity:关灯灵敏度，取值范围0-100，用于控制开灯后亮度变化后的关灯灵敏度； led_brightness_mode：补光灯亮度模式：取值范围0-2（0：自动；1：手动；2：低功耗） led_brightness_value: 补光亮度，取值范围0-100； night_starttime: 夜晚开始时间，用于按照配置的日夜时间来定时开灯，按照时分秒顺序格式化为"%02d:%02d:%02d"； night_endtime: 夜晚结束时间，用于按照配置的日夜时间来定时关灯，按照时分秒顺序格式化为"%02d:%02d:%02d" ; |
| Status |  |

设置灯光配置/HAPI/V1.0/system/light/set

| URL | /HAPI/V1.0/system/light/set |
| --- | --- |
| Description | 设置灯光配置 |
| Method | GET/PUT |
| Sample | 设置灯光控制模式为日夜模式： http://192.168.1.202/HAPI/V1.0/system/light/set?led_ctrl_mode=1&night_endtime=093000&night_starttime=153500&username=admin&password=e10adc3949ba59abbe56e057f20f883e 设置补光灯工作模式为智能双光： http://192.168.1.202/HAPI/V1.0/system/light/set?led_work_mode=2&username=admin&password=e10adc3949ba59abbe56e057f20f883e |
| URL params(简单参数) Input Data(PUT) | 填充以下参数中的一个或者多个参数，缺少参数或者范围不正确将返回错误： led_ctrl_mode: 灯光开关控制模式，取值范围参见2.8.1 led_work_mode: 灯光工作模式，取值范围参见2.8.2； light_open_brightness: 开启环境光亮度，取值范围0-17，值越小说明环境亮度越低才开灯； light_off_sensitivity:关灯灵敏度，取值范围0-100，用于控制开灯后亮度变化后的关灯灵敏度； led_brightness_mode：补光灯亮度模式：取值范围0-2（0：自动；1：手动；2：低功耗） led_brightness_value: 补光亮度，取值范围0-100； night_starttime: 夜晚开始时间，用于按照配置的日夜时间来定时开灯，按照时分秒顺序格式化为"%02d:%02d%:02d"； night_endtime: 夜晚结束时间，用于按照配置的日夜时间来定时关灯，按照时分秒顺序格式化为"%02d:%02d:%02d" ； username/password或者uid |
| Success Response Data | 参见2.8.3GET返回数据。 |
| Note | 返回数据中携带当前灯光配置数据 |
| Status |  |

图像配置

获取图像配置/HAPI/V1.0/system/image/get

| URL | /HAPI/V1.0/system/image/get |
| --- | --- |
| Description | 获取图像当前配置 |
| Method | GET/PUT |
| Sample | http://192.168.1.202/HAPI/V1.0/system/image/get?username=admin&password=e10adc3949ba59abbe56e057f20f883e |
| URL params(简单参数) Input Data(PUT) | username/password或者uid |
| Success Response Data | { "Response": { "ResponseURL": "/HAPI/V1.0/system/image/get", "SessionID": "", "ResponseCode": 0, "ResponseString": "Succeed", "Data": { "brightness": 128, "contrast": 128, "saturation": 128, "sharpness": 128, "tvsystem": 1, "hflip": 0, "vflip": 0, "rotate": 0 } } } |
| Note | brightness：亮度，0-255 contrast：对比度，0-255 saturation：饱和度，0-255 sharpness：锐度：0-255 tvsystem：视频制式 0: NTSC (60HZ) , 1: PAL (50HZ) hflip：水平翻转 vflip：垂直翻转 rotate：走廊模式（旋转90度，部分摄像机支持） |
| Status |  |

设置图像配置/HAPI/V1.0/system/image/set

| URL | /HAPI/V1.0/system/image/set |
| --- | --- |
| Description | 设置图像配置 |
| Method | GET/PUT |
| Sample | 设置亮度： http://192.168.1.202/HAPI/V1.0/system/image/set?brightness=129&username=admin&password=e10adc3949ba59abbe56e057f20f883e 设置视频制式： http://192.168.1.202/HAPI/V1.0/system/image/set?tvsystem=1&username=admin&password=e10adc3949ba59abbe56e057f20f883e |
| URL params(简单参数) Input Data(PUT) | 填充以下参数中的一个或者多个参数，缺少参数或者范围不正确将返回错误： brightness：亮度，0-255 contrast：对比度，0-255 saturation：饱和度，0-255 sharpness：锐度：0-255 tvsystem：视频制式 0: NTSC (60HZ) , 1: PAL (50HZ) hflip：水平翻转 vflip：垂直翻转 rotate：走廊模式（旋转90度，部分摄像机支持）; username/password或者uid |
| Success Response Data | 参见2.9.1 GET返回数据。 |
| Note | 返回数据中携带当前图像配置数据 |
| Status |  |

视频配置

获取视频编码能力集/HAPI/V1.0/system/video/capability

| URL | /HAPI/V1.0/system/video/capability |
| --- | --- |
| Description | 获取支持的视频编码能力集 |
| Method | GET/PUT |
| Sample | http://192.168.1.202/HAPI/V1.0/system/video/capability?username=admin&password=e10adc3949ba59abbe56e057f20f883e |
| URL params(简单参数) Input Data(PUT) | username/password或者uid |
| Success Response Data | { "Response": { "ResponseURL": "/HAPI/V1.0/system/video/capability", "SessionID": "", "ResponseCode": 0, "ResponseString": "Succeed", "Data": [ { "codec_name": "H265+", "res_name": "3840X2160", "stream_type": 0, "def_bitrate": 6000, "min_bitrate": 512, "max_bitrate": 12288, "def_framerate": 20, "min_framerate": 5, "max_framerate": 20, "def_config": 0 }, ………… { "codec_name": "H264", "res_name": "CIF", "stream_type": 2, "def_bitrate": 150, "min_bitrate": 64, "max_bitrate": 2048, "def_framerate": 10, "min_framerate": 5, "max_framerate": 10, "def_config": 0 } ] } } |
| Note | 支持的视频编码列表以json数组返回。数组成员含义： codec_name：编码名称，取值为H264、H265、H265+、MJPEG等 res_name：分辨率名称，常规情况下以分辨率宽X高表示，例如2560X1440，部分常规分辨率用1080P/CIF等字符串直接表示 stream_type：码流序号，0为主码流，1为子码流，2为三码流, def_bitrate：选择该分辨率时的默认码率 min_bitrate：该分辨率支持的最小码率 max_bitrate：该分辨率支持的最大码率 def_framerate：选择该分辨率时的默认帧率 min_framerate：该分辨率支持的最小帧率 max_framerate：该分辨率支持的最大帧率 def_config：该分辨率是否为摄像机的默认配置 |
| Status |  |

获取视频编码配置/HAPI/V1.0/system/video/get

| URL | /HAPI/V1.0/system/video/get |
| --- | --- |
| Description | 获取视频编码配置 |
| Method | GET/PUT |
| Sample | http://192.168.1.202/HAPI/V1.0/system/video/get?username=admin&password=e10adc3949ba59abbe56e057f20f883e |
| URL params(简单参数) Input Data(PUT) | username/password或者uid |
| Success Response Data | { "Response": { "ResponseURL": "/HAPI/V1.0/system/video/get", "SessionID": "", "ResponseCode": 0, "ResponseString": "Succeed", "stream": [ { "enable": 1, "streamID": 1, "encodeFormat": "H265", "resolution": "3840X2160", "bitRateControl": "VBR", "gop": 80, "frameRate": 20, "bitRate": 6000, "bitRateQuality": 0, "qp_enable": 0, "qp_min": 30, "qp_max": 51 }, { "enable": 1, "streamID": 2, "encodeFormat": "H265", "resolution": "640X360", "bitRateControl": "VBR", "gop": 80, "frameRate": 20, "bitRate": 700, "bitRateQuality": 0, "qp_enable": 0, "qp_min": 30, "qp_max": 51 }, { "enable": 0, "streamID": 3, "encodeFormat": "H265", "resolution": "720P", "bitRateControl": "VBR", "gop": 40, "frameRate": 10, "bitRate": 1000, "bitRateQuality": 0, "qp_enable": 0, "qp_min": 28, "qp_max": 51 } ] } } |
| Note | 视频编码配置列表以json数组返回。每个数组成员分别表示主码流、子码流、三码流，数组成员中各元素含义： Enable：使能标记 streamID：码流ID，1为主码流，2为子码流，3为三码流。注意，由于历史原因，这里的数字定义跟/HAPI/V1.0/system/video/capability接口列出的stream_type取值定义不一样。 encodeFormat：编码名称，取值为H264、H265、H265+、MJPEG等 resolution：分辨率名称，常规情况下以分辨率宽X高表示，例如2560X1440，部分常规分辨率用1080P/CIF等字符串直接表示 bitRateControl：码率控制方式，取值为VBR、CBR等 gop：I帧间隔，即每隔多少帧产生一个I帧 frameRate：帧率 bitRate：码率 bitRateQuality：视频质量, 取值范围0-5。0表示自定义，1表示设置为允许的最低码率，2表示设置为最低码率有默认码率的中间值，3表示设置为默认码率，4表示设置为默认码率与最高码率的中间值，5表示设置为允许的最高码率 qp_enable：使能自定义QP qp_min：自定义QP的最小QP值，1-51 qp_max：自定义QP的最大QP值，1-51 |
| Status |  |

设置视频编码配置/HAPI/V1.0/system/video/set

| URL | /HAPI/V1.0/system/video/set |
| --- | --- |
| Description | 设置视频编码 |
| Method | GET/PUT |
| Sample | 设置主码流分辨率： http://192.168.1.202/HAPI/V1.0/system/video/set?streamId=1&resolution=3840X2160&username=admin&password=e10adc3949ba59abbe56e057f20f883e PUT方法： PUT /HAPI/V1.0/system/video/set HTTP/1.1 User-Agent: PostmanRuntime-ApipostRuntime/1.1.0 Cache-Control: no-cache content-type: application/json Accept: */* Accept-Encoding: gzip, deflate, br Connection: keep-alive Host: 192.168.1.202 Content-Length: 116 { "streamId":"1", "resolution":"3840X2160", "username":"admin", "password":"e10adc3949ba59abbe56e057f20f883e" } 设置子码流编码、分辨率、帧率： http://192.168.1.202/HAPI/V1.0/system/video/set?streamId=2&encodeFormat=H264&resolution=VGA&frameRate=25&username=admin&password=e10adc3949ba59abbe56e057f20f883e 设置主码流码率、帧率： 设置播放音量： http://192.168.1.202/HAPI/V1.0/system/audio/set?volume_play=80&username=admin&password=e10adc3949ba59abbe56e057f20f883e |
| URL params(简单参数) Input Data(PUT) | 填充以下参数中的一个或者多个参数，缺少参数或者范围不正确将返回错误： streamID：码流ID，1为主码流，2为子码流，3为三码流。注意，由于历史原因，这里的数字定义跟/HAPI/V1.0/system/video/capability接口列出的stream_type取值定义不一样。 enable：使能标记 encodeFormat：编码名称，取值为H264、H265、H265+、MJPEG等 resolution：分辨率名称，常规情况下以分辨率宽X高表示，例如2560X1440，部分常规分辨率用1080P/CIF等字符串直接表示 bitRateControl：码率控制方式，取值为VBR、CBR等 gop：I帧间隔，即每隔多少帧产生一个I帧 frameRate：帧率 bitRate：码率 bitRateQuality：视频质量, 取值范围0-5。0表示自定义，1表示设置为允许的最低码率，2表示设置为最低码率有默认码率的中间值，3表示设置为默认码率，4表示设置为默认码率与最高码率的中间值，5表示设置为允许的最高码率 qp_enable：使能自定义QP qp_min：自定义QP的最小QP值，1-51 qp_max：自定义QP的最大QP值，1-51 username/password或者uid |
| Success Response Data | 参见2.10.2GET返回数据。 |
| Note | 设置音频参数成功的情况下，返回数据中携带当前音频配置数据 |
| Status |  |

音频配置

获取音频配置编码能力集/HAPI/V1.0/system/audio/capability

| URL | /HAPI/V1.0/system/audio/capability |
| --- | --- |
| Description | 获取支持的音频编码能力集 |
| Method | GET/PUT |
| Sample | http://192.168.1.202/HAPI/V1.0/system/audio/capability?username=admin&password=e10adc3949ba59abbe56e057f20f883e |
| URL params(简单参数) Input Data(PUT) | username/password或者uid |
| Success Response Data | { "Response": { "ResponseURL": "/HAPI/V1.0/system/audio/capability", "SessionID": "", "ResponseCode": 0, "ResponseString": "Succeed", "Data": [ { "codec_name": "G.711", "channels": 1, "bitspersample": 16, "samplerate": 8, "bitrate": 64, "def_config": 1 }, ………… { "codec_name": "AAC", "channels": 2, "bitspersample": 16, "samplerate": 16, "bitrate": 16, "def_config": 0 } ] } } |
| Note | 支持的视频编码列表以json数组返回。数组成员含义： codec_name：编码名称，取值为G.711、G.711A、AAC等 channels：音频通道数 bitspersample ：采样位数，即每个采样占用的比特数 samplerate：采样率 bitrate：比特率 def_config：该编码类型是否为摄像机的默认配置 |
| Status |  |

获取音频配置/HAPI/V1.0/system/audio/get

| URL | /HAPI/V1.0/system/audio/get |
| --- | --- |
| Description | 获取音频配置 |
| Method | GET/PUT |
| Sample | http://192.168.1.202/HAPI/V1.0/system/audio/get?username=admin&password=e10adc3949ba59abbe56e057f20f883e |
| URL params(简单参数) Input Data(PUT) | username/password或者uid |
| Success Response Data | { "Response": { "ResponseURL": "/HAPI/V1.0/system/audio/get", "SessionID": "", "ResponseCode": 0, "ResponseString": "Succeed", "Data": { "enable": 1, "sampleRate": 8000, "encodeFormat": "G.711", "bitRate": 64000, "volume_capture": 90, "volume_play": 95, "amplify": 1, "aec_enable": 1, "mute_ptz_turn": 0 } } } |
| Note | Enable：使能标记 sampleRate：音频编码采样率 encodeFormat：编码名称，编码名称，取值为G.711、G.711A、AAC等 bitRate：音频编码比特率 volume_capture：音频输入音量 volume_play：音频输出音量 amplify：使能设备内部软件功放 aec_enable：使能回声消除（部分设备支持） mute_ptz_turn：使能云台转动时静音功能 |
| Status |  |

设置音频配置/HAPI/V1.0/system/audio/set

| URL | /HAPI/V1.0/system/audio/set |
| --- | --- |
| Description | 获取音频配置 |
| Method | GET/PUT |
| Sample | 设置编码： http://192.168.1.202/HAPI/V1.0/system/audio/set?encodeFormat=G.711&username=admin&password=e10adc3949ba59abbe56e057f20f883e 设置播放音量： http://192.168.1.202/HAPI/V1.0/system/audio/set?volume_play=80&username=admin&password=e10adc3949ba59abbe56e057f20f883e |
| URL params(简单参数) Input Data(PUT) | 填充以下参数中的一个或者多个参数，缺少参数或者范围不正确将返回错误： enable：使能音频编码 sampleRate：音频编码采样率 encodeFormat：编码名称，编码名称，取值为G.711、G.711A、AAC等 bitRate：音频编码比特率 volume_capture：音频输入音量 volume_play：音频输出音量 amplify：使能设备内部软件功放 aec_enable：使能回声消除（部分设备支持） mute_ptz_turn：使能云台转动时静音功能 username/password或者uid |
| Success Response Data | 参见2.11.2GET返回数据。 |
| Note | 设置参数成功的情况下，返回当前配置 |
| Status |  |

OSD配置

获取OSD配置/HAPI/V1.0/system/osd/get

| URL | /HAPI/V1.0/system/osd/get |
| --- | --- |
| Description | 获取视频编码配置 |
| Method | GET/PUT |
| Sample | http://192.168.1.202/HAPI/V1.0/system/osd/get?username=admin&password=e10adc3949ba59abbe56e057f20f883e |
| URL params(简单参数) Input Data(PUT) | username/password或者uid |
| Success Response Data | { "Response": { "ResponseURL": "/HAPI/V1.0/system/osd/get", "SessionID": "", "ResponseCode": 0, "ResponseString": "Succeed", "Data": { "enable": 1, "add_overlay": 3, "style": 3, "DsplayWeek": 1, "OverlayFps": 0, "fontsize": 0, "time24or12": 0, "timeOverlay": { "posType": 0, "posX": 1, "posY": 1, "timeFormat": "yyyy-mm-dd hh:mm:ss" }, "titleOverlay": { "posType": 0, "posX": 0, "posY": 0, "titleType": 0, "title_utf8": "e6b58be8af9543616d657261" } } } } |
| Note | enable：OSD使能 add_overlay：叠加附加信息。0：无，1：叠加分辨率，2：叠加码率，3：叠加分辨率和码率 style：叠加风格，取值范围参考枚举值定义： typedef enum { AJ_OVERLAY_STYLE_BLACK_WHITE = 0, //黑字白底 AJ_OVERLAY_STYLE_WHITE_BLACK = 1, //白字黑底 AJ_OVERLAY_STYLE_TRANSPARENT_BLACKWHITE = 2, //透明背景，黑字白框 AJ_OVERLAY_STYLE_TRANSPARENT_WHITEBLACK = 3, //透明背景，白字黑框 AJ_OVERLAY_STYLE_TRANSPARENT_BLACK = 4, //透明背景，黑字 AJ_OVERLAY_STYLE_TRANSPARENT_WHITE = 5, //透明背景，白字 AJ_OVERLAY_STYLE_INVERSE_COLOR = 6, //反色 }AjOsdOverlayStyle; DsplayWeek:显示星期几 OverlayFps：叠加帧率 fontsize:叠加文字大小，0：标准 1：大字体 2：超大字体 time24or12：0:12小时格式，1：24小时格式 timeOverlay：时间OSD配置 titleOverlay：标题OSD配置 posType：坐标类型，0表示posX、posY配置的是4角位置，1表示posX、posY配置的是画面百分比位置 posX:坐标类型为四角位置时，0画面左边，1表示画面右边，2表示不显示。坐标类型为百分比时，取值范围0-100，表示位置为在画面水平尺寸的百分比。 posY:坐标类型为四角位置时，0画面上边，1表示画面下边，2表示不显示。坐标类型为百分比时，取值范围0-100，表示位置为在画面垂直尺寸的百分比。 timeFormat：时间格式，取值范围： "yyyy-mm-dd hh:mm:ss", "yyyy/mm/dd hh:mm:ss", "yy-mm-dd hh:mm:ss", "yy/mm/dd hh:mm:ss", "hh:mm:ss dd/mm/yyyy", "hh:mm:ss dd-mm-yyyy", "hh:mm:ss mm/dd/yyyy", "hh:mm:ss mm-dd-yyyy", "mm/dd/yyyy hh:mm:ss", "mm-dd-yyyy hh:mm:ss"； titleType：标题类型，0表示叠加文字，1表示叠加图片 title_utf8：当titleType为0时，内容为utf8格式的16进制格式字符串（utf8格式无法直接用ascii码字符串存放和输出，所以将每个字节按16进制格式化成2个字节的ascii字符）；当titleType为1时，内容为摄像机上bmp图片绝对路径。 utf8字符串转换成16进制格式字符串的API参考： int hexdataTohexStr(const char* buf, unsigned int len, char* out, unsigned int outbuflen) { if( outbuflen == 0 ) return -1; if( outbuflen <= 2 ) { out[0] = 0; } if( len >= (outbuflen-1)/2 ) len = (outbuflen-1)/2; unsigned int iIndex; for(iIndex = 0; iIndex < len; iIndex++) { sprintf(&out[2*iIndex], "%02x", (unsigned char)buf[iIndex]); } return 0; } |
| Status |  |

设置OSD配置/HAPI/V1.0/system/osd/set

| URL | /HAPI/V1.0/system/osd/set |
| --- | --- |
| Description | 获取视频编码配置 |
| Method | PUT |
| Sample | PUT /HAPI/V1.0/system/osd/set HTTP/1.1 User-Agent: PostmanRuntime-ApipostRuntime/1.1.0 Cache-Control: no-cache content-type: application/json Accept: */* Accept-Encoding: gzip, deflate, br Connection: keep-alive Host: 192.168.1.202 Content-Length: 404 { "username": "admin", "password": "123456", "enable": 1, "add_overlay": 2, "style": 3, "DsplayWeek": 1, "OverlayFps": 0, "fontsize": 0, "time24or12": 0, "timeOverlay": { "posType": 1, "posX": 50, "posY": 50, "timeFormat": "hh:mm:ss mm-dd-yyyy" }, "titleOverlay": { "posType": 0, "posX": 0, "posY": 0, "titleType": 0, "title_utf8": "43616d657261" } } |
| URL params(简单参数) Input Data(PUT) | 填充以下参数中的一个或者多个参数，缺少参数或者范围不正确将返回错误。参数取值范围参见2.12.1 { "username": "admin", "password": "123456", "enable": 1, "add_overlay": 2, "style": 3, "DsplayWeek": 1, "OverlayFps": 0, "fontsize": 0, "time24or12": 0, "timeOverlay": { "posType": 1, "posX": 50, "posY": 50, "timeFormat": "hh:mm:ss mm-dd-yyyy" }, "titleOverlay": { "posType": 0, "posX": 0, "posY": 0, "titleType": 0, "title_utf8": "43616d657261" } } |
| Success Response Data | 参见2.12.1GET返回数据。 |
| Note | 设置参数成功的情况下，返回当前配置 |
| Status |  |

智能检测配置

获取智能检测能力集/HAPI/V1.0/smart/capability

| URL | /HAPI/V1.0/smart/capability |
| --- | --- |
| Description | 获取智能检测能力集 |
| Method | GET/PUT |
| Sample | http://192.168.1.202/HAPI/V1.0/Smart/capability?username=admin&password=e10adc3949ba59abbe56e057f20f883e |
| URL params(简单参数) Input Data(PUT) | username/password或者uid |
| Success Response Data | { "Response": { "ResponseURL": "/HAPI/V1.0/Smart/capability", "SessionID": "", "ResponseCode": 0, "ResponseString": "Succeed", "Data": { "SupportTargetDetect": 1, "SupportFaceFd": 0, "SupportFaceFr": 0, "SupportLPR": 0, "SupportVG": 0, "SupportRegionAI": 0, "SupportAudioDetect": 1, "SupportCoverDetect": 1 } } } |
| Note | SupportTargetDetect:具备人车非目标检测能力 SupportFaceFd:具备人脸检测能力 SupportFaceFr:具备人脸识别能力 SupportLPR:具备车牌识别能力 SupportVG:具备拌线检测能力 SupportRegionAI:具备区域智能检测（逗留、进入、离开区域等）能力 SupportAudioDetect:支持异常声音检测能力 SupportCoverDetect:支持视频遮挡检测能力 |
| Status |  |

获取目标检测（人车非机动车）算法支持的目标类型/HAPI/V1.0/smart/objectdetect /capability

| URL | /HAPI/V1.0/smart/objectdetect/capability |
| --- | --- |
| Description | 获取智能检测能力集 |
| Method | GET/PUT |
| Sample | http://192.168.1.202/HAPI/V1.0/Smart/objectdetect/capability?username=admin&password=e10adc3949ba59abbe56e057f20f883e |
| URL params(简单参数) Input Data(PUT) | username/password或者uid |
| Success Response Data | { "Response": { "ResponseURL": "/HAPI/V1.0/Smart/objectdetect/capability", "SessionID": "", "ResponseCode": 0, "ResponseString": "Succeed", "Data": [ { "type": 4 } ] } } |
| Note | 支持的目标类型由json数组返回，type定义： typedef enum { AI_TYPE_BIT_CAR = 0, //汽车 AI_TYPE_BIT_MOTO = 1, //摩托车 AI_TYPE_BIT_ELECTRICBICYCLE = 2,//电单车 AI_TYPE_BIT_BICYCLE = 3,//自行车 AI_TYPE_BIT_HUMAN = 4, //人形 AI_TYPE_BIT_FACE = 5, //人脸 AT_TYPE_BIT_NONMOTO_VEHICLE = 6,//非机动车 AT_TYPE_BIT_FIRE=7,//火焰 AT_TYPE_BIT_FALLINGOBJECT=8,//高空抛物 AI_TYPE_BIT_MAX, }AjAiBits; |
| Status |  |

获取目标检测（人车非机动车）算法支持联动能力/HAPI/V1.0/smart/linkage/capability

| URL | /HAPI/V1.0/smart/linkage/capability |
| --- | --- |
| Description | 获取智能检测联动能力集 |
| Method | GET/PUT |
| Sample | http://192.168.1.202/HAPI/V1.0/Smart/linkage/capability?username=admin&password=e10adc3949ba59abbe56e057f20f883e |
| URL params(简单参数) Input Data(PUT) | username/password或者uid |
| Success Response Data | { "Response": { "ResponseURL": "/HAPI/V1.0/Smart/linkage/capability", "SessionID": "", "ResponseCode": 0, "ResponseString": "Succeed", "Data": [ { "SupportLightAction": 0 }, { "SupportBRAlarmAction": 0 }, { "SupportAudioAction": 1 }, { "SupportIOOutAction": 1 } ] } } |
| Note | SupportLightAction：支持补光灯联动闪烁（有检测到目标时闪烁补光灯） SupportBRAlarmAction：支持红蓝警灯联动 SupportAudioAction：支持警戒音联动 SupportIOOutAction：支持IO输出联动 |
| Status |  |

获取警戒音列表/HAPI/V1.0/smart/audiofiles/get

| URL | /HAPI/V1.0/smart/audiofiles/get |
| --- | --- |
| Description | 获取设备警戒音列表 |
| Method | GET/PUT |
| Sample | http://192.168.1.202/HAPI/V1.0/Smart/audiofiles/get?username=admin&password=e10adc3949ba59abbe56e057f20f883e |
| URL params(简单参数) Input Data(PUT) | username/password或者uid |
| Success Response Data | { "Response": { "ResponseURL": "/HAPI/V1.0/Smart/audiofiles/get", "SessionID": "", "ResponseCode": 0, "ResponseString": "Succeed", "Data": [ { "filename": "/opt/ch/attention_en.mp3", "UnicodeDesc": "fffe5700610072006e002000720065006d0069006e006400650072002c00200059006f00750020006800610076006500200065006e0074006500720065006400200074006800650020006d006f006e00690074006f00720069006e00670020006100720065006100" }, { "filename": "/opt/ch/attention03.mp3", "UnicodeDesc": "fffec179ba4e8698df57f78bff526097d18f" }, { "filename": "/opt/ch/attention02.mp3", "UnicodeDesc": "fffea860f25ddb8f6551dd8f81793a53df57f78b3d5ceb5fbb79005f" }, { "filename": "/opt/ch/attention.mp3", "UnicodeDesc": "fffe296ea899d0633a79a860f25ddb8f6551d176a7633a53df57" }, { "filename": "/opt/ch/Audio.mp3" } ] } } |
| Note | Filename为警戒音绝对路径。 UnicodeDesc为相应文件的描述信息，unicode字符串的16进制格式化字符串。 |
| Status |  |

获取运动侦测配置/HAPI/V1.0/motiondetect/get

| URL | /HAPI/V1.0/smart/motiondetect/get |
| --- | --- |
| Description | 获取运动侦测配置 |
| Method | GET/PUT |
| Sample | http://192.168.1.202/HAPI/V1.0/Smart/motiondetect/get?username=admin&password=e10adc3949ba59abbe56e057f20f883e |
| URL params(简单参数) Input Data(PUT) | username/password或者uid |
| Success Response Data | { "Response": { "ResponseURL": "/HAPI/V1.0/Smart/motiondetect/get", "SessionID": "", "ResponseCode": 0, "ResponseString": "Succeed", "Data": { "enable": 1, "ArmingFlag": { "ArmingMode": 1, "WeekPlan": { "day0": 16777215, "day1": 16777215, "day2": 16777215, "day3": 16777215, "day4": 16777215, "day5": 16777215, "day6": 16777215 } }, "AreaConfig": { "block_col": 22, "block_row": 18, "blockconfig": [ { "block_row": "0001111111111111111111" }, { "block_row": "0001111111111111111111" }, { "block_row": "0001111111111111111111" }, { "block_row": "0001111111111111111111" }, { "block_row": "1111111111111111111111" }, { "block_row": "1111111111111111111111" }, { "block_row": "1111111111111111111111" }, { "block_row": "1111111111111111111111" }, { "block_row": "1111111111110000000000" }, { "block_row": "0000000000000000000000" }, { "block_row": "0000000000000000000000" }, { "block_row": "0000000000000000000000" }, { "block_row": "0000000000000000000000" }, { "block_row": "0000000000000000000000" }, { "block_row": "0000000000000000000000" }, { "block_row": "0000000000000000000000" }, { "block_row": "0000000000000000000000" }, { "block_row": "0000000000000000000000" } ] }, "Sensitivity": 80, "AlarmThreshold": 20, "DayNightSwitch": 1, "NightSensitivity": 80, "NightAlarmThreshold": 20, "NightStartTime": "00:00:00", "NightEndTime": "00:00:00", "AlarmAction": { "LightAction": { "ArmingMode": 0, "TimeSpanNum": 0, "DayPlan": [] }, "BRAlarmAction": { "ArmingMode": 0, "TimeSpanNum": 0, "DayPlan": [] }, "AlarmServer": { "ArmingMode": 0, "TimeSpanNum": 0, "DayPlan": [] }, "AlarmPush": { "ArmingMode": 4, "TimeSpanNum": 1, "DayPlan": [ { "startTime": "00:00:00", "endTime": "23:59:59" } ] }, "AudioAction": { "times": 1, "intervalsecnods": 10, "filename": "/opt/ch/attention_en.mp3", "ArmingSetting": { "ArmingMode": 0, "TimeSpanNum": 1, "DayPlan": [ { "startTime": "00:00:00", "endTime": "23:59:59" } ] } }, "IOOutputAction": { "channelCnt": 4, "channels": [ { "portIndex": 1, "enable": 0 }, { "portIndex": 2, "enable": 0 }, { "portIndex": 3, "enable": 0 }, { "portIndex": 4, "enable": 0 } ] } } } } } |
| Note | enable：使能算法检测 ArmingFlag：报警检测布防配置，作为一个总的开关控制所有的联动输出，具体含义参见4.1 AreaConfig：运动侦测区域配置，具体含义参见4.3 Sensitivity:运动侦测灵敏度，0-100。数值越大越容易触发。 AlarmThreshold: 阈值，0-100，配置每个子区域中检测到变化像素的占比阈值，数值越小越容易触发。 DayNightSwitch: 日夜参数开关，启用时在夜晚时使用配置的夜晚灵敏度、阈值 NightSensitivity: 夜晚运动侦测灵敏度 NightAlarmThreshold: 夜晚阈值 NightStartTime: 夜晚开始时间 NightEndTime: 夜晚结束时间 AlarmAction：报警联动配置 LightAction：补光灯联动配置，具体含义参见4.4 BRAlarmAction：红蓝警灯联动配置：具体含义参见4.4 BRAlarmAction：红蓝警灯联动配置：具体含义参见4.4 AlarmServer：联动上报服务器配置：具体含义参见4.4 AlarmPush：报警推送联动配置：具体含义参见4.4 AudioAction：警戒音联动配置：具体含义参见4.5 IOOutputAction：IO输出联动配置：具体含义参见4.6 |
| Status |  |

设置运动侦测配置/HAPI/V1.0/motiondetect/set

| URL | /HAPI/V1.0/smart/motiondetect/set |
| --- | --- |
| Description | 获取运动侦测配置 |
| Method | PUT |
| Sample | PUT /HAPI/V1.0/Smart/motiondetect/set HTTP/1.1 User-Agent: PostmanRuntime-ApipostRuntime/1.1.0 Cache-Control: no-cache Accept: */* Accept-Encoding: gzip, deflate, br Connection: keep-alive Content-Type: application/x-www-form-urlencoded Host: 192.168.1.202 Content-Length: 1175 { "username": "admin", "password": "e10adc3949ba59abbe56e057f20f883e", "AreaConfig": { "block_col": 22, "block_row": 18, "blockconfig": [ { "block_row": "0001111111111111111111" }, { "block_row": "0001111111111111111111" }, { "block_row": "0001111111111111111111" }, { "block_row": "0001111111111111111111" }, { "block_row": "1111111111111111111111" }, { "block_row": "1111111111111111111111" }, { "block_row": "1111111111111111111111" }, { "block_row": "1111111111111111111111" }, { "block_row": "1111111111110000000000" }, { "block_row": "0000000000000000000000" }, { "block_row": "0000000000000000000000" }, { "block_row": "0000000000000000000000" }, { "block_row": "0000000000000000000000" }, { "block_row": "0000000000000000000000" }, { "block_row": "0000000000000000000000" }, { "block_row": "0000000000000000000000" }, { "block_row": "0000000000000000000000" }, { "block_row": "0000000000000000000000" } ] } } |
| URL params(简单参数) Input Data(PUT) | 参考2.13.5获取的JSON数据，填充以下参数中的一个或者多个参数，缺少必要参数或者范围不正确将返回错误。当参数是某项单独配置的JSON结构化数据（ArmingFlag/AreaConfi/以及AlarmAction节点下的LightAction/BRAlarmAction/AlarmServer/AlarmPush/AudioAction/IOOutputAction等）时，需要完整填充所有必须成员参数。 username password enable ArmingFlag AreaConfig Sensitivity AlarmThreshold DayNightSwitch NightSensitivity NightAlarmThreshold NightStartTime NightEndTime AlarmAction |
| Success Response Data | 参见2.13.5GET返回数据。 |
| Note | 设置参数成功的情况下，返回当前配置。 |
| Status |  |

获取目标检测配置/HAPI/V1.0/smart/objectdetect/get

| URL | /HAPI/V1.0/smart/objectdetect/get |
| --- | --- |
| Description | 获取目标检测配置 |
| Method | GET/PUT |
| Sample | http://192.168.1.202/HAPI/V1.0/Smart/objectdetect/get?username=admin&password=e10adc3949ba59abbe56e057f20f883e |
| URL params(简单参数) Input Data(PUT) | username/password或者uid |
| Success Response Data | { "Response": { "ResponseURL": "/HAPI/V1.0/Smart/objectdetect/get", "SessionID": "", "ResponseCode": 0, "ResponseString": "Succeed", "Data": { "enable": 1, "type": { "CAR": 0, "MOTO": 0, "ELECTRICBICYCLE": 0, "BICYCLE": 0, "HUMAN": 1, "FACE": 0, "NONMOTO_VEHICLE": 0, "FIRE": 0, "FALLINGOBJECT": 0 }, "ArmingFlag": { "ArmingMode": 1, "WeekPlan": { "day0": 16777215, "day1": 16777215, "day2": 16777215, "day3": 16777215, "day4": 16777215, "day5": 16777215, "day6": 16777215 } }, "Polygon": { "PointCnt": 7, "Points": [ { "x": 17, "y": 11 }, { "x": 86, "y": 13 }, { "x": 95, "y": 83 }, { "x": 62, "y": 94 }, { "x": 16, "y": 90 }, { "x": 5, "y": 37 }, { "x": 8, "y": 14 } ] }, "Sensitivity": 5, "AlarmThreshold": 75, "minTargetRate": 0, "nonMotionFilter": 1, "AlarmAction": { "LightAction": { "ArmingMode": 0, "TimeSpanNum": 0, "DayPlan": [] }, "BRAlarmAction": { "ArmingMode": 1, "TimeSpanNum": 0, "DayPlan": [] }, "AlarmServer": { "ArmingMode": 0, "TimeSpanNum": 0, "DayPlan": [] }, "AlarmPush": { "ArmingMode": 1, "TimeSpanNum": 0, "DayPlan": [] }, "AudioAction": { "times": 1, "intervalsecnods": 10, "filename": "/opt/ch/attention.mp3", "ArmingSetting": { "ArmingMode": 4, "TimeSpanNum": 3, "DayPlan": [ { "startTime": "01:00:00", "endTime": "13:59:00" }, { "startTime": "15:00:00", "endTime": "17:00:00" }, { "startTime": "21:00:00", "endTime": "22:00:00" } ] } }, "IOOutputAction": { "channelCnt": 4, "channels": [ { "portIndex": 1, "enable": 0 }, { "portIndex": 2, "enable": 0 }, { "portIndex": 3, "enable": 0 }, { "portIndex": 4, "enable": 0 } ] }, "draw_rect_enable": 1, "draw_human_enable": 1, "track_human_enable": 0, "rect_twinkle_enable": 0, "auto_zoom_enable": 0, "gunball_track_mode": 0 } } } } |
| Note | "enable": 算法启用，控制算法的检测，用于实现摄像机的软光敏开关灯等逻辑，以及将报警通知给其他联动模块 "type": 目标检测类型，参见4.7 "ArmingFlag": 算法布防总开关，可配置7X24周计划 "Polygon": 检测区域 "Sensitivity": 灵敏度，取值范围0-10 "AlarmThreshold": 报警阈值（暂未使用） "minTargetRate": 检测最小目标的画面比例，0-100 "nonMotionFilter": 不动不检，0-1 "LightAction": 补光灯联动，有报警时闪烁补光灯 "BRAlarmAction": 红蓝警灯联动，有报警时闪烁专用接口的红蓝警灯 "AlarmServer": 上报到报警中心 "AlarmPush": 推送给APP "AudioAction": 联动警戒声音 "IOOutputAction": 联动IO输出 "draw_rect_enable": 显示检测区域 "draw_human_enable": 显示检测目标 "track_human_enable": 开启人形跟踪 "rect_twinkle_enable": 闪烁检测框 "auto_zoom_enable": 自动变倍跟踪 "gunball_track_mode": 开启枪球跟踪 |
| Status |  |

设置目标检测配置/HAPI/V1.0/smart/objectdetect/set

| URL | /HAPI/V1.0/smart/objectdetect/set |
| --- | --- |
| Description | 设置目标检测配置 |
| Method | PUT |
| Sample | PUT /HAPI/V1.0/Smart/objectdetect/set HTTP/1.1 User-Agent: PostmanRuntime-ApipostRuntime/1.1.0 Cache-Control: no-cache Accept: */* Accept-Encoding: gzip, deflate, br Connection: keep-alive Content-Type: application/x-www-form-urlencoded Host: 192.168.1.202 Content-Length: 294 { "username": "admin", "password": "e10adc3949ba59abbe56e057f20f883e", "ArmingFlag": { "ArmingMode": 2, "WeekPlan": { "day0": 16777211, "day1": 16777212, "day2": 16777213, "day3": 16777214, "day4": 16777215, "day5": 16777216, "day6": 16777217 } } } |
| URL params(简单参数) Input Data(PUT) | 参考2.13.7获取的JSON数据，填充以下参数中的一个或者多个参数，缺少必要参数或者范围不正确将返回错误。当参数是某项单独配置的JSON结构化数据（ArmingFlag/ Polygon/以及AlarmAction节点下的LightAction/BRAlarmAction/AlarmServer/AlarmPush/AudioAction/IOOutputAction等）时，需要完整填充所有必须成员参数。 username password enable type ArmingFlag Polygon Sensitivity AlarmThreshold minTargetRate nonMotionFilter AlarmAction |
| Success Response Data | 参见2.13.7 GET返回数据。 |
| Note | 设置参数成功的情况下，返回当前配置。 |
| Status |  |

获取视频遮挡配置/HAPI/V1.0/smart/videocover/get

| URL | /HAPI/V1.0/smart/videocover/get |
| --- | --- |
| Description | 获取视频遮挡配置 |
| Method | GET/PUT |
| Sample | http://192.168.1.202/HAPI/V1.0/Smart/videocover/get?username=admin&password=e10adc3949ba59abbe56e057f20f883e |
| URL params(简单参数) Input Data(PUT) | username/password或者uid |
| Success Response Data | { "Response": { "ResponseURL": "/HAPI/V1.0/Smart/videocover/get", "SessionID": "", "ResponseCode": 0, "ResponseString": "Succeed", "Data": { "enable": 1, "Sensitivity": 20, "Threadhold_second": 5, "BackgroundUpdateSecond": 40, "ArmingFlag": { "ArmingMode": 1, "WeekPlan": { "day0": 16777215, "day1": 16777215, "day2": 16777215, "day3": 16777215, "day4": 16777215, "day5": 16777215, "day6": 16777215 } }, "AlarmAction": { "AudioAction": { "times": 1, "intervalsecnods": 10, "filename": "/opt/ch/attention_en.mp3", "ArmingSetting": { "ArmingMode": 1, "TimeSpanNum": 1, "DayPlan": [ { "startTime": "00:00:00", "endTime": "23:59:59" } ] } }, "IOOutputAction": { "channelCnt": 4, "channels": [ { "portIndex": 1, "enable": 1 }, { "portIndex": 2, "enable": 0 }, { "portIndex": 3, "enable": 0 }, { "portIndex": 4, "enable": 0 } ] } } } } } |
| Note | 设置参数成功的情况下，返回当前配置 |
| Status |  |

设置视频遮挡配置/HAPI/V1.0/smart/videocover/set

| URL | /HAPI/V1.0/smart/videocover/set |
| --- | --- |
| Description | 设置视频遮挡配置 |
| Method | PUT |
| Sample | PUT /HAPI/V1.0/Smart/videocover/set HTTP/1.1 User-Agent: PostmanRuntime-ApipostRuntime/1.1.0 Cache-Control: no-cache Accept: */* Accept-Encoding: gzip, deflate, br Connection: keep-alive Content-Type: application/x-www-form-urlencoded Host: 192.168.1.202 Content-Length: 91 { "username": "admin", "password": "e10adc3949ba59abbe56e057f20f883e", "enable": 1 } |
| URL params(简单参数) Input Data(PUT) | 参考2.13.9获取的JSON数据，填充以下参数中的一个或者多个参数，缺少必要参数或者范围不正确将返回错误。当参数是某项单独配置的JSON结构化数据时，需要完整填充所有必须成员参数。 username password enable Sensitivity Threadhold_second BackgroundUpdateSecond ArmingFlag Sensitivity AlarmAction |
| Success Response Data | 参见2.13.9 GET返回的数据 |
| Note | 设置参数成功的情况下，返回当前配置 |
| Status |  |

获取人脸检测配置/HAPI/V1.0/smart/facedetect/get

| URL | /HAPI/V1.0/smart/facedetect/get |
| --- | --- |
| Description | 获取人脸检测配置 |
| Method | GET/PUT |
| Sample | http://192.168.1.202/HAPI/V1.0/Smart/facedetect/get?username=admin&password=e10adc3949ba59abbe56e057f20f883e |
| URL params(简单参数) Input Data(PUT) | username/password或者uid |
| Success Response Data | { "Response": { "ResponseURL": "/HAPI/V1.0/Smart/facedetect/get", "SessionID": "", "ResponseCode": 0, "ResponseString": "Succeed", "Data": { "enable": 1, "ArmingFlag": { "ArmingMode": 1, "WeekPlan": { "day0": 16777215, "day1": 16777215, "day2": 16777215, "day3": 16777215, "day4": 16777215, "day5": 16777215, "day6": 16777215 } }, "Area": { "x": 0, "y": 0, "w": 50, "h": 90 }, "Sensitivity": 0, "AlarmThreshold": 75, "AlarmAction": { "draw_rect_enable": 1, "AudioAction": { "times": 1, "intervalsecnods": 0, "filename": "/opt/ch/attention.mp3", "ArmingSetting": { "ArmingMode": 1, "TimeSpanNum": 1, "DayPlan": [ { "startTime": "00:00:00", "endTime": "23:59:59" } ] } }, "IOOutputAction": { "channelCnt": 4, "channels": [ { "portIndex": 1, "enable": 0 }, { "portIndex": 2, "enable": 0 }, { "portIndex": 3, "enable": 0 }, { "portIndex": 4, "enable": 0 } ] } } } } } |
| Note | "enable": 算法启用 "ArmingFlag": 算法布防总开关，可配置7X24周计划 "Area": 检测矩形区域 "Sensitivity": 灵敏度，取值范围0-100 "AlarmThreshold": 检测阈值 "AudioAction": 联动警戒声音 "IOOutputAction": 联动IO输出 "draw_rect_enable": 显示检测区域 |
| Status |  |

设置人脸检测配置/HAPI/V1.0/smart/facedetect/set

| URL | /HAPI/V1.0/smart/facedetect/set |
| --- | --- |
| Description | 设置人脸检测配置 |
| Method | PUT |
| Sample | PUT /HAPI/V1.0/Smart/facedetect/set HTTP/1.1 User-Agent: PostmanRuntime-ApipostRuntime/1.1.0 Cache-Control: no-cache Accept: */* Accept-Encoding: gzip, deflate, br Connection: keep-alive Content-Type: application/x-www-form-urlencoded Host: 192.168.1.202 Content-Length: 1043 { "username": "admin", "password": "e10adc3949ba59abbe56e057f20f883e", "enable": 1, "ArmingFlag": { "ArmingMode": 1, "WeekPlan": { "day0": 16777215, "day1": 16777215, "day2": 16777215, "day3": 16777215, "day4": 16777215, "day5": 16777215, "day6": 16777215 } }, "Area": { "x": 0, "y": 0, "w": 50, "h": 90 }, "Sensitivity": 0, "AlarmThreshold": 75, "AlarmAction": { "draw_rect_enable": 1, "AudioAction": { "times": 1, "intervalsecnods": 0, "filename": "/opt/ch/attention.mp3", "ArmingSetting": { "ArmingMode": 1, "TimeSpanNum": 1, "DayPlan": [ { "startTime": "00:00:00", "endTime": "23:59:59" } ] } }, "IOOutputAction": { "channelCnt": 4, "channels": [ { "portIndex": 1, "enable": 0 }, { "portIndex": 2, "enable": 0 }, { "portIndex": 3, "enable": 0 }, { "portIndex": 4, "enable": 0 } ] } } } |
| URL params(简单参数) Input Data(PUT) | 参考2.13.11获取的JSON数据，填充参数中的一个或者多个参数，缺少必要参数或者范围不正确将返回错误。当参数是某项单独配置的JSON结构化数据时，需要完整填充所有必须成员参数。 |
| Success Response Data | 参考2.13.11获取的JSON数据 |
| Note | 设置参数成功的情况下，返回当前配置 |
| Status |  |

获取越界检测配置/HAPI/V1.0/smart/videogate/get

| URL | /HAPI/V1.0/smart/videogate/get |
| --- | --- |
| Description | 获取越界检测配置 |
| Method | GET/PUT |
| Sample | http://192.168.1.202/HAPI/V1.0/Smart/videogate/get?username=admin&password=e10adc3949ba59abbe56e057f20f883e |
| URL params(简单参数) Input Data(PUT) | username/password或者uid |
| Success Response Data | { "Response": { "ResponseURL": "/HAPI/V1.0/Smart/videogate/get", "SessionID": "", "ResponseCode": 0, "ResponseString": "Succeed", "Data": { "enable": 1, "ArmingFlag": { "ArmingMode": 1, "WeekPlan": { "day0": 16777215, "day1": 16777215, "day2": 16777215, "day3": 16777215, "day4": 16777215, "day5": 16777215, "day6": 16777215 } }, "Rules": [ { "enable": 1, "Sensitivity": 80, "type": { "CAR": 1, "MOTO": 0, "ELECTRICBICYCLE": 0, "BICYCLE": 0, "HUMAN": 0, "FACE": 0, "NONMOTO_VEHICLE": 0, "FIRE": 0, "FALLINGOBJECT": 0 }, "direction": 2, "x0": 0, "y0": 0, "x1": 0, "y1": 100 }, { "enable": 1, "Sensitivity": 0, "type": { "CAR": 0, "MOTO": 0, "ELECTRICBICYCLE": 0, "BICYCLE": 0, "HUMAN": 0, "FACE": 0, "NONMOTO_VEHICLE": 0, "FIRE": 0, "FALLINGOBJECT": 0 }, "direction": 0, "x0": 0, "y0": 0, "x1": 0, "y1": 0 }, { "enable": 0, "Sensitivity": 0, "type": { "CAR": 0, "MOTO": 0, "ELECTRICBICYCLE": 0, "BICYCLE": 0, "HUMAN": 0, "FACE": 0, "NONMOTO_VEHICLE": 0, "FIRE": 0, "FALLINGOBJECT": 0 }, "direction": 0, "x0": 0, "y0": 0, "x1": 0, "y1": 0 }, { "enable": 0, "Sensitivity": 0, "type": { "CAR": 0, "MOTO": 0, "ELECTRICBICYCLE": 0, "BICYCLE": 0, "HUMAN": 0, "FACE": 0, "NONMOTO_VEHICLE": 0, "FIRE": 0, "FALLINGOBJECT": 0 }, "direction": 0, "x0": 0, "y0": 0, "x1": 0, "y1": 0 } ], "AlarmAction": { "LightAction": { "ArmingMode": 0, "TimeSpanNum": 0, "DayPlan": [] }, "BRAlarmAction": { "ArmingMode": 0, "TimeSpanNum": 0, "DayPlan": [] }, "AlarmServer": { "ArmingMode": 0, "TimeSpanNum": 0, "DayPlan": [] }, "AlarmPush": { "ArmingMode": 0, "TimeSpanNum": 0, "DayPlan": [] }, "AudioAction": { "times": 0, "intervalsecnods": 0, "filename": "", "ArmingSetting": { "ArmingMode": 0, "TimeSpanNum": 1, "DayPlan": [ { "startTime": "00:00:00", "endTime": "23:59:59" } ] } }, "IOOutputAction": { "channelCnt": 4, "channels": [ { "portIndex": 1, "enable": 0 }, { "portIndex": 2, "enable": 0 }, { "portIndex": 3, "enable": 0 }, { "portIndex": 4, "enable": 0 } ] }, "draw_rect_enable": 0, "draw_target_enable": 0 } } } } |
| Note | "enable": 算法启用 "ArmingFlag": 算法布防总开关，可配置7X24周计划 "Rules": 配置检测规则，最多可以配置4条，参见4.10 "type": 目标检测类型，参见4.7 "LightAction": 补光灯联动，有报警时闪烁补光灯 "BRAlarmAction": 红蓝警灯联动，有报警时闪烁专用接口的红蓝警灯 "AlarmServer": 上报到报警中心 "AlarmPush": 推送给APP "AudioAction": 联动警戒声音 "IOOutputAction": 联动IO输出 "draw_rect_enable": 显示检测区域 "draw_target_enable": 显示检测目标 |
| Status |  |

设置越界检测配置/HAPI/V1.0/smart/videogate/set

| URL | /HAPI/V1.0/smart/videogate/set |
| --- | --- |
| Description | 设置越界检测配置 |
| Method | PUT |
| Sample | PUT /HAPI/V1.0/Smart/videogate/set HTTP/1.1 User-Agent: PostmanRuntime-ApipostRuntime/1.1.0 Cache-Control: no-cache Accept: */* Accept-Encoding: gzip, deflate, br Connection: keep-alive Content-Type: application/x-www-form-urlencoded Host: 192.168.1.202 Content-Length: 476 { "username": "admin", "password": "e10adc3949ba59abbe56e057f20f883e", "enable": 1, "ArmingFlag": { "ArmingMode": 1 }, "Rules": [ { "enable": 1, "Sensitivity": 80, "type": { "CAR": 1, "HUMAN": 1 }, "direction": 2, "x0": 0, "y0": 0, "x1": 100, "y1": 100 } ] } |
| URL params(简单参数) Input Data(PUT) | 参考2.13.13获取的JSON数据，填充参数中的一个或者多个参数，缺少必要参数或者范围不正确将返回错误。当参数是某项单独配置的JSON结构化数据时，需要完整填充所有必须成员参数。 |
| Success Response Data | 参考2.13.13获取的JSON数据 |
| Note | 设置参数成功的情况下，返回当前配置 |
| Status |  |

获取区域(周界)侦测配置/HAPI/V1.0/smart/regionai/get

| URL | /HAPI/V1.0/smart/videogate/get |
| --- | --- |
| Description | 获取区域(周界)侦测配置 |
| Method | GET/PUT |
| Sample | http://192.168.1.202/HAPI/V1.0/Smart/regionai/get?username=admin&password=e10adc3949ba59abbe56e057f20f883e |
| URL params(简单参数) Input Data(PUT) | username/password或者uid |
| Success Response Data | { "Response": { "ResponseURL": "/HAPI/V1.0/Smart/regionai/get", "SessionID": "", "ResponseCode": 0, "ResponseString": "Succeed", "Data": { "enable": 1, "ArmingFlag": { "ArmingMode": 1, "WeekPlan": { "day0": 16777215, "day1": 16777215, "day2": 16777215, "day3": 16777215, "day4": 16777215, "day5": 16777215, "day6": 16777215 } }, "Polygon": { "PointCnt": 4, "Points": [ { "x": 0, "y": 0 }, { "x": 100, "y": 0 }, { "x": 100, "y": 100 }, { "x": 0, "y": 100 } ] }, "Rules": [ { "enable": 1, "Sensitivity": 80, "type": { "CAR": 1, "MOTO": 0, "ELECTRICBICYCLE": 0, "BICYCLE": 0, "HUMAN": 1, "FACE": 0, "NONMOTO_VEHICLE": 0, "FIRE": 0, "FALLINGOBJECT": 0 }, "mode": "LEAVE", "stayseconds": 5 }, { "enable": 0, "Sensitivity": 0, "type": { "CAR": 0, "MOTO": 0, "ELECTRICBICYCLE": 0, "BICYCLE": 0, "HUMAN": 0, "FACE": 0, "NONMOTO_VEHICLE": 0, "FIRE": 0, "FALLINGOBJECT": 0 }, "mode": "STAY", "stayseconds": 0 }, { "enable": 0, "Sensitivity": 0, "type": { "CAR": 0, "MOTO": 0, "ELECTRICBICYCLE": 0, "BICYCLE": 0, "HUMAN": 0, "FACE": 0, "NONMOTO_VEHICLE": 0, "FIRE": 0, "FALLINGOBJECT": 0 }, "mode": "STAY", "stayseconds": 0 } ], "AlarmAction": { "LightAction": { "ArmingMode": 0, "TimeSpanNum": 0, "DayPlan": [] }, "BRAlarmAction": { "ArmingMode": 0, "TimeSpanNum": 0, "DayPlan": [] }, "AlarmServer": { "ArmingMode": 0, "TimeSpanNum": 0, "DayPlan": [] }, "AlarmPush": { "ArmingMode": 0, "TimeSpanNum": 0, "DayPlan": [] }, "AudioAction": { "times": 0, "intervalsecnods": 0, "filename": "", "ArmingSetting": { "ArmingMode": 0, "TimeSpanNum": 1, "DayPlan": [ { "startTime": "00:00:00", "endTime": "23:59:59" } ] } }, "IOOutputAction": { "channelCnt": 4, "channels": [ { "portIndex": 1, "enable": 0 }, { "portIndex": 2, "enable": 0 }, { "portIndex": 3, "enable": 0 }, { "portIndex": 4, "enable": 0 } ] }, "draw_rect_enable": 0, "draw_target_enable": 0 } } } } |
| Note | "enable": 算法启用 "ArmingFlag": 算法布防总开关，可配置7X24周计划 "Polygon": 检测区域 "Rules": 配置检测规则，最多可以配置3条，参见4.11 "type": 目标检测类型，参见4.7 "LightAction": 补光灯联动，有报警时闪烁补光灯 "BRAlarmAction": 红蓝警灯联动，有报警时闪烁专用接口的红蓝警灯 "AlarmServer": 上报到报警中心 "AlarmPush": 推送给APP "AudioAction": 联动警戒声音 "IOOutputAction": 联动IO输出 "draw_rect_enable": 显示检测区域 "draw_target_enable": 显示检测目标 |
| Status |  |

设置区域(周界)侦测配置/HAPI/V1.0/smart/regionai/set

| URL | /HAPI/V1.0/smart/regionai/set |
| --- | --- |
| Description | 设置区域(周界)侦测配置 |
| Method | PUT |
| Sample | PUT /HAPI/V1.0/Smart/regionai/set HTTP/1.1 User-Agent: PostmanRuntime-ApipostRuntime/1.1.0 Cache-Control: no-cache Accept: */* Accept-Encoding: gzip, deflate, br Connection: keep-alive Content-Type: application/x-www-form-urlencoded Host: 192.168.1.202 Content-Length: 659 { "username": "admin", "password": "e10adc3949ba59abbe56e057f20f883e", "enable": 1, "ArmingFlag": { "ArmingMode": 1 }, "Polygon": { "PointCnt": 4, "Points": [ { "x": 0, "y": 0 }, { "x": 100, "y": 0 }, { "x": 100, "y": 100 }, { "x": 0, "y": 100 } ] }, "Rules": [ { "enable": 1, "Sensitivity": 80, "type": { "CAR": 1, "MOTO": 0, "ELECTRICBICYCLE": 0, "BICYCLE": 0, "HUMAN": 1, "FACE": 0, "NONMOTO_VEHICLE": 0, "FIRE": 0, "FALLINGOBJECT": 0 }, "mode": "LEAVE", "stayseconds": 5 } ] } |
| URL params(简单参数) Input Data(PUT) | 参考2.13.15获取的JSON数据，填充参数中的一个或者多个参数，缺少必要参数或者范围不正确将返回错误。当参数是某项单独配置的JSON结构化数据时，需要完整填充所有必须成员参数。 |
| Success Response Data | 参考2.13.15获取的JSON数据 |
| Note | 设置参数成功的情况下，返回当前配置 |
| Status |  |

获取车牌识别配置/HAPI/V1.0/smart/lpr/get

| URL | /HAPI/V1.0/smart/lpr/get |
| --- | --- |
| Description | 获取车牌识别配置 |
| Method | GET/PUT |
| Sample | http://192.168.1.202/HAPI/V1.0/Smart/lpr/get?username=admin&password=e10adc3949ba59abbe56e057f20f883e |
| URL params(简单参数) Input Data(PUT) | username/password或者uid |
| Success Response Data | { "Response": { "ResponseURL": "/HAPI/V1.0/Smart/lpr/get", "SessionID": "", "ResponseCode": 0, "ResponseString": "Succeed", "Data": { "enable": 1, "Sensitivity": 80, "detectionmode": 0, "actionInterval": 5, "snapQuality": "Default", "ArmingFlag": { "ArmingMode": 1, "WeekPlan": { "day0": 16777215, "day1": 16777215, "day2": 16777215, "day3": 16777215, "day4": 16777215, "day5": 16777215, "day6": 16777215 } }, "Polygon": { "PointCnt": 4, "Points": [ { "x": 0, "y": 0 }, { "x": 100, "y": 0 }, { "x": 100, "y": 100 }, { "x": 0, "y": 100 } ] }, "AlarmAction": { "AlarmServer": { "ArmingMode": 1, "TimeSpanNum": 0, "DayPlan": [] }, "AlarmPush": { "ArmingMode": 1, "TimeSpanNum": 0, "DayPlan": [] }, "draw_rect_enable": 1, "draw_target_enable": 1, "draw_osd_enable": 1, "play_voice_enable": 1 } } } } |
| Note | "enable": 算法启用，控制算法的检测，用于实现摄像机的软光敏开关灯等逻辑，以及将报警通知给其他联动模块 "ArmingFlag": 算法布防总开关，可配置7X24周计划 "Polygon": 检测区域 "Sensitivity": 灵敏度，取值范围0-100 "detectionmode": 推图模式，0:最优推图 1:间隔推图 "actionInterval":相同车牌的情况下两次上报时间间隔，单位秒 "snapQuality ": 抓图质量(分辨率)，Default,1280X720,后续根据需要增加其他分辨率支持 "draw_rect_enable": 绘制检测区域 "draw_target_enable": 绘制检测目标 "draw_osd_enable": 绘制识别到的车牌 "play_voice_enable": 语音播报车牌 |
| Status |  |

设置车牌识别配置/HAPI/V1.0/smart/lpr/set

| URL | /HAPI/V1.0/smart/lpr/set |
| --- | --- |
| Description | 设置车牌识别配置 |
| Method | PUT |
| Sample | PUT /HAPI/V1.0/Smart/lpr/set HTTP/1.1 User-Agent: PostmanRuntime-ApipostRuntime/1.1.0 Cache-Control: no-cache Accept: */* Accept-Encoding: gzip, deflate, br Connection: keep-alive Content-Type: application/x-www-form-urlencoded Host: 192.168.1.202 Content-Length: 663 { "username": "admin", "password": "e10adc3949ba59abbe56e057f20f883e", "enable": 1, "Sensitivity": 80, "detectionmode": 0, "actionInterval": 5, "snapQuality": "Default", "ArmingFlag": { "ArmingMode": 1 }, "Polygon": { "PointCnt": 4, "Points": [ { "x": 0, "y": 0 }, { "x": 100, "y": 0 }, { "x": 100, "y": 100 }, { "x": 0, "y": 100 } ] }, "AlarmAction": { "AlarmServer": { "ArmingMode": 1 }, "AlarmPush": { "ArmingMode": 1 }, "draw_rect_enable": 1, "draw_target_enable": 1, "draw_osd_enable": 1, "play_voice_enable": 1 } } |
| URL params(简单参数) Input Data(PUT) | 参考2.13.17获取的JSON数据，填充参数中的一个或者多个参数，缺少必要参数或者范围不正确将返回错误。当参数是某项单独配置的JSON结构化数据时，需要完整填充所有必须成员参数。 |
| Success Response Data | 参考2.13.17获取的JSON数据 |
| Note | 设置参数成功的情况下，返回当前配置。 |
| Status |  |

获取烟火检测配置/HAPI/V1.0/smart/flameflumes/get

| URL | /HAPI/V1.0/smart/flameflumes/get |
| --- | --- |
| Description | 获取烟火配置 |
| Method | GET/PUT |
| Sample | http://192.168.1.202/HAPI/V1.0/Smart/flameflumes/get?username=admin&password=e10adc3949ba59abbe56e057f20f883e |
| URL params(简单参数) Input Data(PUT) | username/password或者uid |
| Success Response Data | { "Response": { "ResponseURL": "/HAPI/V1.0/Smart/flameflumes/get", "SessionID": "", "ResponseCode": 0, "ResponseString": "Succeed", "Data": { "enable": 1, "Sensitivity": 60, "actionInterval": 5, "snapQuality": "1280X720", "ArmingFlag": { "ArmingMode": 0, "WeekPlan": { "day0": 16777215, "day1": 16777215, "day2": 16777215, "day3": 16777215, "day4": 16777215, "day5": 16777215, "day6": 16777215 } }, "Polygon": { "PointCnt": 4, "Points": [ { "x": 0, "y": 0 }, { "x": 100, "y": 0 }, { "x": 100, "y": 100 }, { "x": 0, "y": 100 } ] }, "AlarmAction": { "LightAction": { "ArmingMode": 0, "TimeSpanNum": 0, "DayPlan": [] }, "BRAlarmAction": { "ArmingMode": 0, "TimeSpanNum": 0, "DayPlan": [] }, "AlarmServer": { "ArmingMode": 1, "TimeSpanNum": 0, "DayPlan": [] }, "AlarmPush": { "ArmingMode": 0, "TimeSpanNum": 0, "DayPlan": [] }, "AudioAction": { "times": 1, "intervalsecnods": 0, "filename": "/opt/ch/Welcome.mp3", "ArmingSetting": { "ArmingMode": 0, "TimeSpanNum": 1, "DayPlan": [ { "startTime": "00:00:00", "endTime": "23:59:59" } ] } }, "IOOutputAction": { "channelCnt": 4, "channels": [ { "portIndex": 1, "enable": 0 }, { "portIndex": 2, "enable": 0 }, { "portIndex": 3, "enable": 0 }, { "portIndex": 4, "enable": 0 } ] }, "draw_rect_enable": 1, "draw_target_enable": 1 } } } } |
| Note | "enable": 算法启用，控制算法的检测，用于实现摄像机的软光敏开关灯等逻辑，以及将报警通知给其他联动模块 "ArmingFlag": 算法布防总开关，可配置7X24周计划 "Polygon": 检测区域 "Sensitivity": 灵敏度，取值范围0-100 "actionInterval":相同车牌的情况下两次上报时间间隔，单位秒 "snapQuality ": 抓图质量(分辨率)，Default,1280X720,后续根据需要增加其他分辨率支持 "draw_rect_enable": 绘制检测区域 "draw_target_enable": 绘制检测目标 "LightAction": 补光灯联动，有报警时闪烁补光灯 "BRAlarmAction": 红蓝警灯联动，有报警时闪烁专用接口的红蓝警灯 "AlarmServer": 上报到报警中心 "AlarmPush": 推送给APP "AudioAction": 联动警戒声音 "IOOutputAction": 联动IO输出 |
| Status |  |

设置烟火检测配置/HAPI/V1.0/smart/flameflumes/set

| URL | /HAPI/V1.0/smart/flameflumes/set |
| --- | --- |
| Description | 设置烟火检测配置 |
| Method | PUT |
| Sample | PUT /HAPI/V1.0/Smart/flameflumes/set HTTP/1.1 User-Agent: PostmanRuntime-ApipostRuntime/1.1.0 Cache-Control: no-cache Accept: */* Accept-Encoding: gzip, deflate, br Connection: keep-alive Content-Type: application/x-www-form-urlencoded Host: 192.168.1.202 Content-Length: 452 { "username": "admin", "password": "e10adc3949ba59abbe56e057f20f883e", "enable": 1, "Sensitivity": 60, "actionInterval": 5, "snapQuality": "1280X720", "ArmingFlag": { "ArmingMode": 0 }, "AlarmAction": { "AlarmServer": { "ArmingMode": 1 }, "AlarmPush": { "ArmingMode": 0 }, "AudioAction": { "ArmingSetting": { "ArmingMode": 0 } }, "draw_rect_enable": 1, "draw_target_enable": 1 } } |
| URL params(简单参数) Input Data(PUT) | 参考2.13.19获取的JSON数据，填充参数中的一个或者多个参数，缺少必要参数或者范围不正确将返回错误。当参数是某项单独配置的JSON结构化数据时，需要完整填充所有必须成员参数。 |
| Success Response Data | 参考2.13.19获取的JSON数据 |
| Note | 设置参数成功的情况下，返回当前配置。 |
| Status |  |

事件

事件工作流程

流程说明

告警上报

步骤1：客户端开启TCP服务用于接受服务端事件推送

步骤2：客户端向设备侧（服务端）订阅告警，接口详情参见注册订阅/HAPI/V1.0/Event/subscription/regist

步骤3：设备侧（服务端）返回的参数分别代表订阅标识（ID）、订阅开始时间（CurrentTime）及结束时间（TerminationTime）等订阅信息，订阅标识（ID）用于后续告警的刷新或取消。

步骤4：客户端需在订阅结束时间（TerminationTime）之前定期发送刷新接口进行刷新，刷新订阅。

步骤5：设备侧（服务端）返回刷新的订阅时间。

步骤6：如果设备侧（服务端）发生告警，设备侧通过TCP短连接向客户端指定的地址推送事件，详见事件通知。事件通知遵循HTTP协议，通过JSON携带事件信息。

告警取消

步骤1：客户端取消告警，接口详情参见删除订阅/HAPI/V1.0/Event/subscription/delete

步骤2：服务端取消成功，不再推送告警。

事件订阅

注册订阅/HAPI/V1.0/Event/subscription/regist

| URL | /HAPI/V1.0/Event/subscription/regist |
| --- | --- |
| Description | 注册订阅 |
| Method | POST |
| Sample | POST /HAPI/V1.0/Event/subscription/regist?username=admin&password=e10adc3949ba59abbe56e057f20f883e HTTP/1.1 User-Agent: PostmanRuntime-ApipostRuntime/1.1.0 Cache-Control: no-cache Accept: */* Accept-Encoding: gzip, deflate, br Connection: keep-alive Content-Type: application/x-www-form-urlencoded Host: 192.168.1.202 Content-Length: 107 { "ServerType":0, "ServerName": "192.168.1.253", "Port": 9998, "Duration": 3600, "EventType": "all" } |
| URL params(简单参数) Input Data(POST) | { "ServerType": 0, "ServerName": "192.168.1.253", "Port": 9998, "Duration": 3600, "PostURLPrefix":"", "EventType": "all" } |
| Success Response Data | { "Response": { "ResponseURL": "/HAPI/V1.0/Event/subscription/regist", "SessionID": "", "ResponseCode": 0, "ResponseString": "Succeed", "Data": { "ID": 2, "ServerType": 9998, "ServerName": "192.168.1.253", "ServerPort": 9998, "EventType": "all", "CurrentTime": 1699930618, "TerminationTime": 1699934218 } } } |
| Note | 设备上对相同服务器和端口的注册订阅请求，不会创建的新的重复订阅。 |
| Status |  |

| Param | Requirment | Type | Description | Example |
| --- | --- | --- | --- | --- |
| ServerType | M | Enum | IP地址类型： 0: IPv4； 1:域名 | 0 |
| ServerName | M | string | IPv4地址或者域名。 长度范围为[0,64]。 |  |
| Port | M | unsigned long | 端口，范围为[1, 65535]。 |  |
| Duration | M | unsigned long | 订阅周期，单位为s，范围为[30, 3600]。 |  |
| PostURLPrefix | C | string | 设备上报event时需要添加的URL前缀，用以报警事件服务器指定路径。 | 0 |
| EventType | C | string | 订阅事件类型，不填或者填”all”时意味着需要上报所有事件，不同事件用逗号分割。 | "MotionDetect,ObjectDetect” |
| ID | M | unsigned long | 设备返回的订阅标识，用以识别哪次订阅，刷新订阅、删除订阅操作需要携带此ID，以指明需操作哪次订阅。 | 0 |
| CurrentTime | M | unsigned long | UTC时间，从1970年1月1日0点开始的秒数。 | 1477104900 |
| TerminationTime | M | unsigned long | UTC时间，从1970年1月1日0点开始的秒数。 | 1477104949 |

刷新订阅/HAPI/V1.0/Event/subscription/refresh

| URL | /HAPI/V1.0/Event/subscription/refresh |
| --- | --- |
| Description | 刷新订阅 |
| Method | POST |
| Sample | POST /HAPI/V1.0/Event/subscription/refresh?username=admin&password=e10adc3949ba59abbe56e057f20f883e HTTP/1.1 User-Agent: PostmanRuntime-ApipostRuntime/1.1.0 Cache-Control: no-cache Accept: */* Accept-Encoding: gzip, deflate, br Connection: keep-alive Content-Type: application/x-www-form-urlencoded Host: 192.168.1.202 Content-Length: 40 { "ID": 1, "Duration": 3600 } |
| URL params(简单参数) Input Data(POST) | { "ID": 2, "Duration": 3600 } |
| Success Response Data | { "Response": { "ResponseURL": "/HAPI/V1.0/Event/subscription/refresh", "SessionID": "", "ResponseCode": 0, "ResponseString": "Succeed", "Data": { "ID": 2, "ServerType": 9998, "ServerName": "192.168.1.253", "ServerPort": 9998, "EventType": "all", "CurrentTime": 1699931648, "TerminationTime": 1699935248 } } } |
| Note | 参数定义参见3.2.1 |
| Status |  |

删除订阅/HAPI/V1.0/Event/subscription/delete

| URL | /HAPI/V1.0/Event/subscription/delete |
| --- | --- |
| Description | 删除订阅 |
| Method | POST |
| Sample | POST /HAPI/V1.0/Event/subscription/delete?username=admin&password=e10adc3949ba59abbe56e057f20f883e HTTP/1.1 User-Agent: PostmanRuntime-ApipostRuntime/1.1.0 Cache-Control: no-cache Accept: */* Accept-Encoding: gzip, deflate, br Connection: keep-alive Content-Type: application/x-www-form-urlencoded Host: 192.168.1.202 Content-Length: 17 { "ID": 2 } |
| URL params(简单参数) Input Data(POST) | { "ID": 2 } |
| Success Response Data | { "Response": { "ResponseURL": "/HAPI/V1.0/Event/subscription/delete", "SessionID": "", "ResponseCode": 0, "ResponseString": "Succeed", "Data": { "ID": 2 } } } |
| Note | 参数定义参见3.2.1 |
| Status |  |

事件通知

事件通知/HAPI/V1.0/Event/Notification

| URL | /HAPI/V1.0/Event/Notification |
| --- | --- |
| Description | 推送事件通知。 |
| Method | POST |
| Input Data | { "AlarmType":, "AlarmSubType":, "TimeStamp": "ChannelNo": "NotificationType":, "DeviceID":, "Picture":, "AlarmInfo": } |
| Success Return Data | None |
| Note |  |
| Status |  |

| AlarmType | M | string | 告警类型 参见表格 1 AlarmType/AlarmSubType事件类型 | "ObjectDetect" |
| --- | --- | --- | --- | --- |
| AlarmSubType | M | string | 告警类型 参见表格 1 AlarmType/AlarmSubType事件类型 | "HumanShapeDetect" |
| TimeStamp | M | unsigned long | 告警时间。 UTC时间，从1970年1月1日0点开始的秒数。 | 1489040894 |
| Channel | M | unsigned long | 告警源通道ID。 单通道产品固定填0； 多通道产品0表示通道无关的事件；通道事件填通道ID号（从1开始）。 | 1 |
| NotificationType | M | unsigned long | 通知类型： 0：实时通知 1：历史通知 | 0 |
| OccurFlag | M | String | 报警发生标记： “true”:报警发生 “false”:报警结束 | true |
| DeviceID | M | string | 告警设备ID，填写设备序列号，或者其他报警服务器要求的定制标识符 | “EF000000000000A3” |
| Picture | C | Json Block | jpg图片信息，携带图片size和Data，Data为对jpg进行base64编码后的字符串 |  |
| AlarmInfo | M | Json Block | 根据不同的AlarmType，携带不同的结构化数据信息，以及图片信息等 | - |

表格 1 AlarmType/AlarmSubType事件类型

| AlarmType/事件类型 | AlarmSubType/事件子类型 | 事件描述 |
| --- | --- | --- |
| MotiDetect | MotiAlarm | 运动检测告警 |
| ObjectDetect | HumanShapeDetect | 人形检测报警 |
|  | VehicleDetect | 车辆检测告警 |
|  | FallingObjectsDetectiAlarm | 高空抛物报警 |
|  | ElectricbicycleDetect | 电单车检测 |
|  | HumanShapeDetect | 人形检测报警消失 |
|  | VehicleDetect | 车辆检测告警消失 |
|  | FallingObjectsDetectiAlarm | 高空抛物报警消失 |
|  | ElectricbicycleDetect | 电单车检测报警消失 |
| MaskDetect | MaskImageAlarm | 遮挡侦测告警 |
| AudioDetect | AbnormalAudio | 音频异常检测告警 |
| CrosslineDetect | LineDetectorCrossed | 越界告警 |
| RegiDetect | EnterArea | 进入区域 |
|  | LeaveArea | 离开区域 |
|  | Loitering | 区域徘徊 |
|  | LoiteringDetecti | 徘徊检测 |
|  | FenceCrossing | 翻越围栏 |
|  | ObjectRemoved | 物品看护 |
|  | ObjectLeftBehind | 物品遗留 |
|  | PeopleGathering | 人员聚集 |
|  | AreaPeopleCountingAlarm | 区域人数统计告警 |
| FireDetect | FireAlarm | 火焰告警 |
|  | FumesAlarm | 烟雾告警 |
|  | FlameAndFumesAlarm | 烟火告警 |
| FaceDetect | FaceIsDetected | 人脸检测 |
|  | FaceRecogniti | 人脸比对报警 |
|  | FaceMatchAlarm | 人脸识别匹配报警 |
|  | FaceRecognitiMatchlistAlarm | 人脸识别匹配报警 |
|  | FaceNotMatchAlarm | 人脸识别不匹配报警 |
|  | FaceRecognitiMismatchlistAlarm | 人脸识别不匹配报警 |
| LprDetect | LprDetectAlarm | 车牌识别报警 |
|  | LprMatchlistAlarm | 车牌识别匹配报警 |
|  | LprMismatchlistAlarm | 车牌识别不匹配报警 |
|  | LprBlacklistAlarm | 停车场车辆识别黑名单报警 |
| IllegalLogin | IllegalLogin | 非法访问 |
| BatteryDetect | LowBattery | 低电量报警 |
| NetworkDetect | NetworkDiscnected | 网络断开异常告警 |
|  | IPCflict | IP冲突异常告警 |
| IOInputDetect | InputAlarm | 输入开关量告警 |
| TemperatureDetect | TemperatureDetectiAlarm | 温度检测事件 |
|  | HighTemperatureHighAlarm | 温度过高报警 |
|  | TemperatureLowAlarm | 温度过低报警 |
|  | TemperatureAbnormalAlarm | 温度异常报警 |
| HumanStatusDetect | SafetyHelmetAlarm | 未佩戴安全帽子报警 |
|  | TelephingAlarm | 打电话报警 |
|  | SmokingAlarm | 吸烟告警 |
|  | BodyTemperatureAlarm | 体温异常告警 |
|  | NoMaskAlarm | 未戴口罩告警 |

移动侦测报警发生消息示例：

POST /HAPI/V1.0/Event/Notification HTTP/1.1

User-Agent: ANJVISION HAPI NOTIFY

Cache-Control: no-cache

Connection: keep-alive

Content-Type: application/x-www-form-urlencoded

x-cos-meta-User-ID: 1

Host: 192.168.1.253

Content-Length: 983

{"AlarmType":"MotionDetect","AlarmSubType":"MotionAlarm","TimeStamp":1705310501,"ChannelNo":0,"NotificationType":0,"OccurFlag":"true","DeviceID":"EF000000000000A3","AlarmInfo":{"image":null,"MotionDetectInfo":{"motion_col":22,"motion_row":18,"motion_details":[{"motion_row":"0000000000000000000000"},{"motion_row":"0000000000000000000000"},{"motion_row":"0000000000000000000000"},{"motion_row":"0000000000000000000000"},{"motion_row":"0000000000000000000000"},{"motion_row":"0000000000000000000000"},{"motion_row":"0000000000000000000000"},{"motion_row":"0000000000000000000000"},{"motion_row":"0000000100000000000000"},{"motion_row":"0000000000000000000000"},{"motion_row":"0100000000000000000000"},{"motion_row":"0000000000000000000000"},{"motion_row":"0000000000000000000000"},{"motion_row":"0000000000000000000000"},{"motion_row":"0000000000000000000000"},{"motion_row":"0000000000000000000000"},{"motion_row":"0000000000000000000000"},{"motion_row":"0000000000000000000000"}]}}}

移动侦测报警结束消息示例：

POST /HAPI/V1.0/Event/Notification HTTP/1.1

User-Agent: ANJVISION HAPI NOTIFY

Cache-Control: no-cache

Connection: keep-alive

Content-Type: application/x-www-form-urlencoded

x-cos-meta-User-ID: 1

Host: 192.168.1.253

Content-Length: 182

{"AlarmType":"MotionDetect","AlarmSubType":"MotionAlarm","TimeStamp":1705310512,"ChannelNo":0,"NotificationType":0,"OccurFlag":"false","DeviceID":"EF000000000000A3","AlarmInfo":null}

JSON结构化数据说明

布防方式（ArmingMode）

布防方式可以配置为：禁用、全天启用、白天启用、夜晚启用、自定义时间段。定义如下：

typedef enum

{

ARMING_DISABLE = 0,//不布防

ARMING_ALLDAY = 1,//全天布防

ARMING_DAYTIME = 2,//白天布防

ARMING_NIGHT = 3,//夜晚布防

ARMING_CUSTOM = 4,//自定义时间段

}ArmingMode;

当布防方式为自定义时间段时，报警检测布防配置根据周计划来触发各种报警联动；具体某种报警联动布防配置根据日计划来触发联动。

报警检测布防配置

报警检测布防配置，区别于报警检测启用开关，主要用于控制所有的报警联动输出。也就是说，报警检测启用的情况下，可以按照布防计划来启用或者统一关闭所有的报警联动输出。在报警检测布防计划时间段内的时候，每种报警联动控制再根据各自独立的布防计划去输出。

报警检测布防方式定义参见4.1。当配置为自定义时间段时，按照7X24小时配置布防周计划表，最小时间单位为1小时。JSON数据表示时，每天用1个32位整数来表示，每小时占用1个bit来。

JSON数据：

"ArmingFlag": {

"ArmingMode": 1,

"WeekPlan": {

"day0": 16777215,

"day1": 16777215,

"day2": 16777215,

"day3": 16777215,

"day4": 16777215,

"day5": 16777215,

"day6": 16777215

}

}

运动侦测区域配置

运动侦测区域按照最大18行22列的子区域来配置，每个子区域用字符’0’和’1’来表示启用该区域的运动侦测。

JSON数据中，区域配置使用JSON字符串数组来表示。

"AreaConfig": {

"block_col": 22,

"block_row": 18,

"blockconfig": [

{

"block_row": "0001111111111111111111"

},

{

"block_row": "0001111111111111111111"

},

{

"block_row": "0001111111111111111111"

},

{

"block_row": "0001111111111111111111"

},

{

"block_row": "1111111111111111111111"

},

{

"block_row": "1111111111111111111111"

},

{

"block_row": "1111111111111111111111"

},

{

"block_row": "1111111111111111111111"

},

{

"block_row": "1111111111110000000000"

},

{

"block_row": "0000000000000000000000"

},

{

"block_row": "0000000000000000000000"

},

{

"block_row": "0000000000000000000000"

},

{

"block_row": "0000000000000000000000"

},

{

"block_row": "0000000000000000000000"

},

{

"block_row": "0000000000000000000000"

},

{

"block_row": "0000000000000000000000"

},

{

"block_row": "0000000000000000000000"

},

{

"block_row": "0000000000000000000000"

}

]

},

报警联动布防配置

报警联动布防方式定义参见4.1，用于配置联动日计划，当配置为自定义时间段时，配置一日内最多4个时间段

JSON数据：

{

"ArmingMode": 4,

"TimeSpanNum": 1,

"DayPlan": [

{

"startTime": "00:00:00",

"endTime": "23:59:59"

}

]

}

警戒音联动配置

警戒音联动用于配置报警触发时设备播放的提示音，以及警戒音联动布防计划。

JSON数据：

"AudioAction": {

"times": 1,

"intervalsecnods": 10,

"filename": "/opt/ch/attention_en.mp3",

"ArmingSetting": {

"ArmingMode": 0,

"TimeSpanNum": 1,

"DayPlan": [

{

"startTime": "00:00:00",

"endTime": "23:59:59"

}

]

}

}

IO输出联动配置

IO输出联动配置用于配置报警触发时联动控制的IO输出列表。IO输出的布防计划，由IO输出配置来决定。

JSON数据：

"IOOutputAction": {

"channelCnt": 4,

"channels": [

{

"portIndex": 1,

"enable": 0

},

{

"portIndex": 2,

"enable": 0

},

{

"portIndex": 3,

"enable": 0

},

{

"portIndex": 4,

"enable": 0

}

]

}

目标检测类型

目标检测类型用于配置目标检测算法中的目标类型，目前主要使用人形和车型。

JSON数据：

"type": {

"CAR": 0,

"MOTO": 0,

"ELECTRICBICYCLE": 0,

"BICYCLE": 0,

"HUMAN": 1,

"FACE": 0,

"NONMOTO_VEHICLE": 0,

"FIRE": 0,

"FALLINGOBJECT": 0

}

目标检测区域配置

目标检测区域配置用于配置目标检测的区域范围，支持最多10个顶点的不规则多边形区域。每个顶点的使用屏幕百分比x/y坐标来表示，不能有线段交叉。

JSON数据：

"Polygon": {

"PointCnt": 7,

"Points": [

{

"x": 17,

"y": 11

},

{

"x": 86,

"y": 13

},

{

"x": 95,

"y": 83

},

{

"x": 62,

"y": 94

},

{

"x": 16,

"y": 90

},

{

"x": 5,

"y": 37

},

{

"x": 8,

"y": 14

}

]

}

矩形检测区域配置

矩形检测区域配置用于配置人脸检测等区域范围，配置左上顶点位置以及矩形宽、高，使用屏幕百分比x/y坐标来表示。

JSON数据：

"Area": {

"x": 0,

"y": 0,

"w": 50,

"h": 90

}

越界检测规则配置

越界检测最多配置4条规则。type参见4.7.。每条规则配置线条的两个端点（AB）坐标位置，使用屏幕百分比x/y坐标来表示。direction 取值范围：0: A<->B 1: A->B 2: A<-B (SIDE A MEANS LEFT OF THE LINE)

JSON数据：

"Rules": [

{

"enable": 1,

"Sensitivity": 80,

"type": 1,

"direction": 2,

"x0": 50,

"y0": 0,

"x1": 50,

"y1": 100

},

{

"enable": 1,

"Sensitivity": 50,

"type": 1,

"direction": 0,

"x0": 30,

"y0": 10,

"x1": 70,

"y1": 90

},

{

"enable": 0,

"Sensitivity": 0,

"type": 0,

"direction": 0,

"x0": 0,

"y0": 0,

"x1": 0,

"y1": 0

},

{

"enable": 0,

"Sensitivity": 0,

"type": 0,

"direction": 0,

"x0": 0,

"y0": 0,

"x1": 0,

"y1": 0

}

],

区域侦测规则配置

越界检测最多配置3条规则。type参见4.7.。每条规则检测类型、检测模式（STAY:停留，ENTER:进入，LEAVE：离开）。模式为STAY的时候，需要配置stayseconds（报警等停留时间）。

JSON数据：

"Rules": [

{

"enable": 1,

"Sensitivity": 80,

"type": {

"CAR": 1,

"MOTO": 0,

"ELECTRICBICYCLE": 0,

"BICYCLE": 0,

"HUMAN": 1,

"FACE": 0,

"NONMOTO_VEHICLE": 0,

"FIRE": 0,

"FALLINGOBJECT": 0

},

"mode": "STAY",

"stayseconds": 0

},

{

"enable": 0,

"Sensitivity": 0,

"type": {

"CAR": 0,

"MOTO": 0,

"ELECTRICBICYCLE": 0,

"BICYCLE": 0,

"HUMAN": 0,

"FACE": 0,

"NONMOTO_VEHICLE": 0,

"FIRE": 0,

"FALLINGOBJECT": 0

},

"mode": "STAY",

"stayseconds": 0

},

{

"enable": 0,

"Sensitivity": 0,

"type": {

"CAR": 0,

"MOTO": 0,

"ELECTRICBICYCLE": 0,

"BICYCLE": 0,

"HUMAN": 0,

"FACE": 0,

"NONMOTO_VEHICLE": 0,

"FIRE": 0,

"FALLINGOBJECT": 0

},

"mode": "STAY",

"stayseconds": 0

}

],

事件通知Json Block

MotionDetectInfo

{

"motion_col": 22,

"motion_row": 18,

"motion_details": [

{

"motion_row": "0000000011111111111111"

},

{

"motion_row": "0000000010111111111111"

},

{

"motion_row": "0000000010111111111111"

},

{

"motion_row": "0000111010011111111111"

},

{

"motion_row": "0111110111111111111111"

},

{

"motion_row": "1110011111111111111111"

},

{

"motion_row": "1111110100111111111111"

},

{

"motion_row": "0111000000011111111111"

},

{

"motion_row": "1111001011110000000000"

},

{

"motion_row": "0000000000000000000000"

},

{

"motion_row": "0000000000000000000000"

},

{

"motion_row": "0000000000000000000000"

},

{

"motion_row": "0000000000000000000000"

},

{

"motion_row": "0000000000000000000000"

},

{

"motion_row": "0000000000000000000000"

},

{

"motion_row": "0000000000000000000000"

},

{

"motion_row": "0000000000000000000000"

},

{

"motion_row": "0000000000000000000000"

}

]

}

| Param | Requirement | Type | Description | Example |
| --- | --- | --- | --- | --- |
| motion_col | M | unsigned long | 图像宽度分割列数 | 22 |
| motion_row | M | unsigned long | 图像高度分割行数 | 18 |
| motion_details | M | array | 每一行子区域运动检测结果，用字符’0’表示该区域没有检测到运动，’1’来表示该区域检测到运动 |  |

    - ObjectDetectInfo

暂空，后续根据算法完善